STREET REGULATIONS 33-8-8

CHAPTER 33
STREET REGULATIONS

ARTICLE I – DEPARTMENT ESTABLISHED


33-1-1
DEPARTMENT ESTABLISHED.  There is hereby established a Department of the municipal government which shall be known as the Street Department.  It shall embrace the Street Committee, the Superintendent, and the employees.  The Village Engineer shall serve as ex-officio officer.


33-1-2
COMMITTEE ON STREETS.  The Village Board Standing Committee on Streets shall exercise a general supervision over the affairs of the Street Department.  It shall ascertain the needs and conditions thereof and shall, from time to time, report the same to the Mayor and Village Board.

ARTICLE II - GENERAL REGULATIONS


33-2-1
UNDERMINING.  No person shall undermine in any manner, any street or any other ground or real estate situated in the Village or belonging to any private person.


33-2-2
OPEN DOORS.  No person shall open or allow to remain open, any door, any gate, or the grating of any vault belonging to the premises occupied by him, on any street, alley or sidewalk in the Village for any purpose, except the taking in and removing goods; and any person allowing such grating to remain open shall warn passersby of the danger.


33-2-3 
REPAIRING SIDEWALKS, ETC.  Whenever any sidewalk, pavement, or cellar door on the same becomes worn out or out of repair, or is torn up or broken and uneven, it shall be the duty of the Street Superintendent to immediately report such fact to the Mayor or Street and Alley Committee, and upon verbal or written order from either of them, to give notice to the party owning the adjoining property to repair such sidewalk or cellar door without delay.


33-2-4 
STAIRWAY - RAILING.  Steps or stairways leading into any building shall not extend from the wall of such building onto any pavement or sidewalk, and in such case, the person making or causing to be made such passage shall erect a railing on the side of the stairs toward the street to prevent persons from falling into the street.


33-2-5 
CLOSING STREET.  Whenever public safety or the improvement or repair of any street, alley or public place requires it, the Mayor may order any street, alley, or public place temporarily closed to traffic and the placing of signs indicating that the street, alley or public place is closed by order of the Mayor.  Whenever such signs are so placed, no person shall ride or drive upon or cross such street, alley or public place, or in any manner, destroy, deface, or remove any such sign.


33-2-6 
SIGNS ACROSS STREET.  No person shall place any sign, advertisement or banner over any or across any street, alley or sidewalk in the Village, unless he has written approval of the Village Board. (See 65 ILCS Sec. 5/11-80-17)

33-2-7 
VEHICLES AND SKATEBOARDS ON SIDEWALKS.  No person shall operate any skateboard or motor vehicle on or over any sidewalk, except in crossing the same to go into a yard or parking lot.


33-2-8 
DEPOSITS ON SIDEWALKS AND STREETS.  It shall be unlawful to deposit on any public sidewalk, any material which may be harmful to the pavement thereof, or any waste material, or any glass or other articles which might cause injury to persons, animals or property.


Merchandise or other articles may be deposited on sidewalks preparatory to delivery, provided that the usable width is not thereby reduced to less than four feet (4'); and provided that no such article shall remain on such walk for more than thirty (30) minutes.

33-2-9
OBSTRUCTING STREET.


(A)

It shall be unlawful to deposit any material on any street which may be harmful to the pavement thereof, or any waste material, or any grass clippings, or to cause a lawn mower to blow grass clippings onto a street or any other articles such as glass which may cause injury to any person, animal or property.


(B)

No person shall place or cause to be placed or erected on any public ground, or in any public street, alley or sidewalk in the Village, any debris, materials, or obstruction, except as may be permitted by this Code.


(C)

It shall be the duty of the Police Department to exercise a vigilant supervision over such places and to notify any person found making such deposit or responsible for same to remove the offending matter at once.  (See 65 ILCS Sec. 5/11-80-3)  (Ord. No. 10-09; 12-06-10)

33-2-10
BUILDING MATERIALS IN STREET.  The Superintendent may move any obstruction on any street or sidewalk of the Village, but before doing so, he shall notify the person responsible therefore to remove such obstruction within a reasonable time after being notified.  Any person engaged in erecting a building or fence or improving any lot on such street may deposit materials thereon and contiguous to such length of time as may be necessary for the work.  The obstruction shall not extend to more than one-half (1/2) of the 

width of the sidewalk, street, or alley adjacent to such improvement and the gutter shall always be left free and unobstructed.  At night, such person shall keep an illuminated warning light on such material.  (See 65 ILCS Sec. 5/11-80-3) (Ord. No. 10-09; 12-06-10 in part)

33-2-11
MERCHANDISE ON PUBLIC STREET.  It shall be unlawful for any person, firm or corporation to use any street, sidewalk, or other public place as space for the display of goods or merchandise for sale; or to write or make any signs or advertisements on any such pavements, unless permission is granted by the Village Board.  (See 65 ILCS Sec. 5/11-80-3)


33-2-12
RAINWATER DRAINS.  It shall be unlawful to construct or permit the construction of any storm water drain or any drainage pipe in either a natural or man-made ditch without having first obtained a permit therefor.  Applications for such permits shall be made to the Village Clerk and shall be accompanied by a statement as to the purpose of such drainage pipe, the premises to be served and the specification of such pipe to be installed.  Such application shall be referred to the Street Superintendent and no such permit shall be issued unless he shall have found that the Village Code would be complied with by the installation of such storm water drain or drainage pipe and, that the installation of such storm water drain or drainage pipe would not interfere with, overload, obstruct or otherwise adversely affect the existing storm water drainage system within the Village.


It shall be unlawful to construct or permit the construction of any storm water drain which discharges water onto any sidewalk in the Village and it shall be unlawful to construct or permit the maintenance of any such drain which discharges into any public street or alley at a height greater than eighteen (18) inches above the ground or pavement.


33-2-13
ENCROACHMENTS.  It shall be unlawful to erect or maintain any building or structure which encroaches upon any public street or property.  (Ord. No. 78-3; 07-03-78 in part)

33-2-14
POSTING BILLS.  It shall be unlawful for any person to paste, paint, print or nail any handbill, sign, poster, advertisement or notice of any kind on any curbstone, flagstone, or any other portion or part of any sidewalk, or upon any tree, lamppost, utility pole, hydrant, or upon any private wall, door or gate without the consent, in writing, of the owner of such curbstone, flagstone, sidewalk, tree, lamppost, utility pole, hydrant, private wall, door or gate.


33-2-15
SIGNS ON POLES.  No person shall nail, tack, paste, paint or fasten, or cause to be nailed, tacked, painted or fastened, any sign or any other foreign substance or material onto any telephone, telegraph, electric light, police and/or fire alarm pole or post, or any street or traffic sign located on any sidewalk, street, alley or public grounds or injure or deface any such pole or post.


33-2-16
INJURY TO NEW PAVEMENTS.  It shall be unlawful to walk upon or drive any vehicle or animal upon or destroy any newly-laid sidewalk pavement while the same is guarded by a warning sign or barricade, or to knowingly injure any soft, newly-laid pavement.


33-2-17
BARBED-WIRE FENCES.  It shall be unlawful to maintain or construct any fence composed in whole or in part of barbed wire, or with any similar material designed to cause injury to persons, or any wire charged with electrical current, anywhere within three feet (3’) of any public street, sidewalk, alley, park or other public way or place unless such barbs or charged wire are at least eight feet (8’) above the level of such public place.


33-2-18
BURNING ON PUBLIC STREETS.  It shall be unlawful for any person to burn any leaves, paper, rubbish or other substances upon any of the public streets, sidewalks or alleys in the Village.


33-2-19
GRASS MOWING.  Property owners and/or their tenants shall be jointly and separately responsible for mowing the grass or weeds between the property lines and the adjoining street surfaces.  The height of the grass or weeds shall not exceed eight (8) inches.

ARTICLE III - TREES AND SHRUBS


33-3-1
PLANTING.  It shall be unlawful to plant any tree or bush in any public street or parkway or other public place without having first secured a permit therefore.  Applications for such permits shall be made to the Street Superintendent and shall be referred by him to the Village Board.  All trees and shrubs so planted shall be placed subject to the directions and approval of the Village Board.


33-3-2
PLANTING TREES IN RIGHT-OF-WAY.  It shall be unlawful to plant any bushes, trees, shrubs or other plants on the right-of-way of any public street, including the space on the right-of-way between the sidewalk and the adjacent street pavement.


33-3-3
REMOVAL.  It shall be unlawful to remove or cut down any tree or shrub or portion thereof in any street, parkway or other public place without having first secured a permit therefore.  Applications for such permits shall be made to the Street Superintendent and shall be referred by him to the Village board before permission shall be granted.


33-3-4
INJURY.  It shall be unlawful to injure any tree or shrub planted in such public place.


33-3-5
ADVERTISEMENTS OR NOTICES.  It shall be unlawful to attach any sign, advertisement or notice to any tree or shrub in any street, parkway, or other public place.


33-3-6
DANGEROUS TREES.  Any tree or shrub which overhangs any sidewalk, street or other public place in the municipality at a height less than eight (8) feet or in such a way as to impede or interfere with traffic or travel on such public place shall be trimmed by the owner of the abutting premises or of the premises on which such tree or shrub grows so that the obstruction shall cease.


Any tree or limb of a tree which has become likely to fall on or across any public way or place shall be removed by the owner of the premises on which such tree grows or stands.


The Street Superintendent may, at the owner’s expense, trim or remove any tree or shrub so that the obstruction or danger to traffic or passage shall be done away with.


33-3-7
WIRES.  It shall be unlawful to attach any wires or rope to any tree or shrub in any public street, parkway or other public place without the permission of the Village Board.


Any person or company given the right to maintain the poles and wires in the streets, alleys, or other public places in the municipality shall, in the absence of provision in the franchise concerning the subject, keep such wires and poles free from and away from any trees 

or shrubs in such places so far as may be possible and shall keep all such trees and shrubs properly trimmed, subject to the supervision of the Street Superintendent so that no injury shall be done either to the poles or wires or the shrubs and trees by their conduct.


33-3-8
GAS PIPES.  Any person or company maintaining any gas pipe in the municipality shall, in the absence of provision in the franchise concerning the subject, keep such pipes free from leaks so that no injury shall be done to any trees or shrubs.

(See 65 ILCS 5/11-80-2)

ARTICLE IV

CONSTRUCTION OF UTILITY FACILITIES IN THE RIGHTS-OF-WAY


33-4-1
PURPOSE AND SCOPE.


(A)

Purpose.  The purpose of this Article is to establish policies and procedures for constructing facilities on rights-of-way within the Village’s jurisdiction, which will provide public benefit consistent with the preservation of the integrity, safe usage, and visual qualities of the Village rights-of-way and the Village as a whole.


(B)

Intent.  In enacting this Article, the Village intends to exercise its authority over the rights-of-way in the Village and, in particular, the use of the public ways and property by utilities, by establishing uniform standards to address issues presented by utility facilities, including without limitation:

(1)
prevent interference with the use of streets, sidewalks, alleys, parkways and other public ways and places;

(2)
prevent the creation of visual and physical obstructions and other conditions that are hazardous to vehicular and pedestrian traffic;

(3)
prevent interference with the facilities and operations of the Village’s utilities and of other utilities lawfully located in rights-of-way or public property;

(4)
protect against environmental damage, including damage to trees, from the installation of utility facilities;

(5)
protect against increased stormwater run-off due to structures and materials that increase impermeable surfaces;

(6)
preserve the character of the neighborhoods in which facilities are installed;

(7)
preserve open space, particularly the tree-lined parkways that characterize the Village’s residential neighborhoods;

(8)
prevent visual blight from the proliferation of facilities in the rights-of-way; and

(9)
assure the continued safe use and enjoyment of private properties adjacent to utility facilities locations.


(C)

Facilities Subject to this Article.  This Article applies to all facilities on, over, above, along, upon, under, across, or within the rights-of-way within the jurisdiction of the Village.  A facility lawfully established prior to the effective date of this Article may continue to be maintained, repaired and operated by the utility as presently constructed and located, except as may be otherwise provided in any applicable franchise, license or similar agreement.


(D)

Franchises, Licenses, or Similar Agreements.  The Village, in its discretion and as limited by law, may require utilities to enter into a franchise, license or similar agreement for the privilege of locating their facilities on, over, above, along, upon, under, across, or within the Village rights-of-way.  Utilities that are not required by law to enter into such an agreement may request that the Village enter into such an agreement.  In such an agreement, the Village may provide for terms and conditions inconsistent with this Article.


(E)

Effect of Franchises, Licenses, or Similar Agreements.  

(1)
Utilities Other Than Telecommunications Providers.  In the event that a utility other than a telecommunications provider has a franchise, license or similar agreement with the Village, such franchise, license or similar agreement shall govern and control during the term of such agreement and any lawful renewal or extension thereof.

(2)
Telecommunications Providers.  In the event of any conflict with, or inconsistency between, the provisions of this Article and the provisions of any franchise, license or similar agreement between the Village and any telecommunications provider, the provisions of such franchise, license or similar agreement shall govern and control during the term of such agreement and any lawful renewal or extension thereof.


(F)

Conflicts With Other Articles or Chapters.  This Article supersedes all Articles or Chapters or parts of Articles or Chapters adopted prior hereto that are in conflict herewith, to the extent of such conflict.


(G)

Conflicts With State and Federal Laws.  In the event that applicable federal or State laws or regulations conflict with the requirements of this Article, the utility shall comply with the requirements of this Article to the maximum extent possible without violating federal or State laws or regulations.


(H)

Sound Engineering Judgment.  The Village shall use sound engineering judgment when administering this Article and may vary the standards, conditions, and requirements expressed in this Article when the Village so determines.  Nothing herein shall be construed to limit the ability of the Village to regulate its rights-of-way for the protection of the public health, safety and welfare.


33-4-2
DEFINITIONS.  As used in this Article and unless the context clearly requires otherwise, the words and terms listed shall have the meanings ascribed to them in this Section.  Any term not defined in this Section shall have the meaning ascribed to it in 92 Ill. Adm. Code. § 520.30, unless the context clearly requires otherwise.


“AASHTO”:  American Association of State Highway and Transportation Officials.


“ANSI”:  American National Standards Institute.


“Applicant”:  A person applying for a permit under this Article.


“ASTM”:  American Society for Testing Materials.


“Backfill”:  The methods or materials for replacing excavated material in a trench or pit.


“Bore” or “Boring”:  To excavate an underground cylindrical cavity for the insertion of a pipe or electrical conductor.


“Cable Operator”:  That term as defined in 47 U.S.C. 522(5).


“Cable Service”:  That tern as defined in 47 U.S.C. 522(6).


“Cable System”:  That term as defined in 47 U.S.C. 522(7).


“Carrier Pipe”:  The pipe enclosing the liquid, gas or slurry to be transported.


“Casing”:  A structural protective enclosure for transmittal devices such as: carrier pipes, electrical conductors, and fiber optic devices.


“Clear Zone”:  The total roadside border area, starting at the edge of the pavement, available for safe use by errant vehicles.  This area may consist of a shoulder, a recoverable 

slope, a non-recoverable slope, and a clear run-out area.  The desired width is dependent upon the traffic volumes and speeds, and on the roadside geometry.  Distances are specified in the AASHTO Roadside Design Guide.


“Coating”:  Protective wrapping or mastic cover applied to buried pipe for protection against external corrosion.


“Conductor”:  Wire carrying electric current.


“Conduit”:  A casing or encasement for wires or cables.


“Construction” or “Construct”:  The installation, repair, maintenance, placement, alteration, enlargement, demolition, modification or abandonment in place of facilities.


“Cover”:  The depth of earth or backfill over buried utility pipe or conductor.


“Crossing Facility”:  A facility that crosses one or more right-of-way lines of a right-of-way.


“Disrupt the Right-of-Way”:  For the purposes of this Article, any work that obstructs the right-of-way or causes a material adverse affect on the use of the right-of-way for its intended use.  Such work may include, without limitation, the following: excavating or other cutting; placement (whether temporary or permanent) of materials, equipment, devices, or structures; damage to vegetation; and compaction or loosening of the soil, and shall not include the parking of vehicles or equipment in a manner that does not materially obstruct the flow of traffic on a highway.


“Emergency”:  Any immediate maintenance to the facility required for the safety of the public using or in the vicinity of the right-of-way or immediate maintenance required for the health and safety of the general public served by the utility.


“Encasement”:  Provision of a protective casing.


“Engineer”:  The Village Engineer or his or her designee.


“Equipment”:  Materials, tools, implements, supplies, and/or other items used to facilitate construction of facilities.


“Excavation”:  The making of a hole or cavity by removing material, or laying bare by digging.


“Extra Heavy Pipe”:  Pipe meeting ASTM standards for this pipe designation.


“Facility”:  All structures, devices, objects, and materials (including, but not limited to, track and rails, wires, ducts, fiber optic cable, antennas, vaults, boxes, equipment enclosures, cabinets, pedestals, poles, conduits, grates, covers, pipes, cables, and appurtenances thereto) located on, over, above, along, upon, under, across, or within rights-of-way under this Article.  For purposes of this Article, the term “facility” shall not include any facility owned or operated by the Village.


“Freestanding Facility”:  A facility that is not a crossing facility or a parallel facility, such as an antenna, transformer, pump, or meter station.


“Frontage Road”:  Roadway, usually parallel, providing access to land adjacent to the highway where it is precluded by control of access on highway.


“Hazardous Materials”:  Any substance or material which, due to its quantity, form, concentration, location, or other characteristics, is determined by the Village Engineer or Superintendent to pose an unreasonable and imminent risk to the life, health or safety of persons or property or to the ecological balance of the environment, including, but not limited to explosives, radioactive materials, petroleum or petroleum products or gases, poisons, etiology (biological) agents, flammables, corrosives or any substance determined to be hazardous or toxic under any federal or state law, statute or regulation.


“Highway Code”:  The Illinois Highway Code, 605 ILCS 5/1-101 et seq., as amended from time to time.


“Highway”:  A specific type of right-of-way used for vehicular traffic including rural or urban roads, alleys or streets.  “Highway” includes all highway land and improvements, including roadways, ditches and embankments, bridges, drainage structures, sign, guardrails, protective structures and appurtenances necessary or convenient for vehicle traffic.


“Holder”:  A person or entity that has received authorization to offer or provide cable or video service from the ICC pursuant to the Illinois Cable and Video Competition Law, 220 ILCS 5/21-401.


“IDOT”:  Illinois Department of Transportation.


“ICC”:  Illinois Commerce Commission.


“Jacking”:  Pushing a pipe horizontally under a roadway by mechanical means with or without boring.


“Jetting”:  Pushing a pipe through the earth using water under pressure to create a cavity ahead of the pipe.


“Joint Use”:  The use of pole lines, trenches or other facilities by two or more utilities.


“Major Intersection”:  The intersection of two or more major arterial highways.


“Occupancy”:  The presence of facilities on, over or under right-of-way.


“Parallel Facility”:  A facility that is generally parallel or longitudinal to the centerline of a right-of-way.


“Parkway”:  Any portion of the right-of-way not improved by street or sidewalk.


“Pavement Cut”:  The removal of an area of pavement for access to facility or for the construction of a facility.


“Permittee”:  That entity to which a permit has been issued pursuant to Sections 33-4-4 and 33-4-5 of this Article.


“Practicable”:  That which is performable, feasible or possible, rather than that which is simply convenient.


“Pressure”:  The internal force acting radically against the walls of a carrier pipe expressed in pounds per square inch gauge (psig).


“Petroleum Products Pipelines”:  Pipelines carrying crude or refined liquid petroleum products including, but not limited to, gasoline, distillates, propane, butane, or coal-slurry.


“Prompt”:  That which is done within a period of time specified by the Village.  If no time period is specified, the period shall be thirty (30) days.


“Public Entity”:  A legal entity that constitutes or is part of the government, whether at local, state or federal level.


“Restoration”:  The repair of a right-of-way, highway, roadway, or other area disrupted by the construction of a facility.


“Right-of-Way” or “Rights-of-Way”:  Any street, alley, other land or waterway, dedicated or commonly used for pedestrian or vehicular traffic or other similar purposes, including utility easements, in which the Village has the right and authority to authorize, regulate or permit the location of facilities other than those of the Village.  “Right-of-way” or “rights-of-way” shall not include any real or personal Village property that is not specifically described in the previous two sentences and shall not include Village buildings, fixtures and other structures or improvements, regardless of whether they are situated in the right-of-way.


“Roadway”:  That part of the highway that includes the pavement and shoulders.


“Sale of Telecommunications at Retail”:  The transmitting, supplying, or furnishing of telecommunications and all services rendered in connection therewith for a consideration, other than between a parent corporation and its wholly owned subsidiaries or between wholly owned subsidiaries, when the gross charge made by one such corporation to another such 

corporation is not greater than the gross charge paid to the retailer for their use or consumption and not for sale.


“Security Fund”:  That amount of security required pursuant to Section 33-4-10.


“Shoulder”:  A width of roadway, adjacent to the pavement, providing lateral support to the pavement edge and providing an area for emergency vehicular stops and storage of snow removed from the pavement.


“Sound Engineering Judgment”:  A decision(s) consistent with generally accepted engineering principles, practices and experience.


“Superintendent of Public Works”:  The Superintendent of Public Works or his or her designee, hereinafter referred to as “Superintendent”.


“Telecommunications”:  This term includes, but is not limited to, messages or information transmitted through use of local, toll, and wide area telephone service, channel services, telegraph services, teletypewriter service, computer exchange service, private line services, mobile radio services, cellular mobile telecommunications services, stationary two-way radio, paging service and any other form of mobile or portable one-way or two-way communications, and any other transmission of messages or information by electronic or similar means, between or among points by wire, cable, fiber optics, laser, microwave, radio, satellite, or similar facilities.  “Private line” means a dedicated non-traffic sensitive service for a single customer that entitles the customer to exclusive or priority use of a communications channel, or a group of such channels, from one or more specified locations to one or more other specified locations.  “Telecommunications” shall not include value added services in which computer processing applications are used to act on the form, content, code and protocol of the information for purposes other than transmission.  “Telecommunications” shall not include purchase of telecommunications by a telecommunications service provider for use as a component part of the service provided by such provider to the ultimate retail consumer who originates or terminates the end-to-end communications.  “Telecommunications” shall not include the provision of cable services through a cable system as defined in the Cable Communications Act of 1984 (47 U.S.C. Sections 521 and following), as now or hereafter amended, or cable or other programming services subject to an open video system fee payable to the Village through an open video system as defined in the Rules of the Federal Communications Commission (47 C.F.R. §76.1500 and following), as now or hereafter amended.


“Telecommunications Provider”:  Means any person that installs, owns, operates or controls facilities in the right-of-way used or designed to be used to transmit telecommunications in any form.


“Telecommunications Retailer”:  Means and includes every person engaged in making sales of telecommunication at retail as defined herein.


“Trench”:  A relatively narrow open excavation for the installation of an underground facility.


“Utility”:  The individual or entity owning or operating any facility as defined in this Article.


“Vent”:  A pipe to allow the dissipation into the atmosphere of gases or vapors from an underground casing.


“Video Service”:  That term as defined in Section 21-201(v) of the Illinois Cable and Video Competition Law of 2007, 220 ILCS 21-201(v).


“Village”:  The Village of Mansfield.


“Water Lines”:  Pipelines carrying raw or potable water.


“Wet Boring”:  Boring using water under pressure at the cutting auger to soften the earth and to provide a sluice for the excavated material.


33-4-3
ANNUAL REGISTRATION REQUIRED.  Every utility that occupies right-of-way within the Village shall register on January 1 of each year with the Superintendent, providing the utility’s name, address and regular business telephone and telecopy numbers, the name of one or more contact persons who can act on behalf of the utility in connection with emergencies involving the utility’s facilities in the right-of-way and a twenty-four (24) hour telephone number for each such person, and evidence of insurance as required in Section 33-4-8 of this Article, in the form of a certificate of insurance.


33-4-4
PERMIT REQUIRED; APPLICATIONS AND FEES.


(A)

Permit Required.  No person shall construct (as defined in this Article) any facility on, over, above, along, upon, under, across, or within any Village right-of-way which:

(1)
changes the location of the facility;

(2)
adds a new facility;

(3)
disrupts the right-of-way (as defined in this Article), or

(4)
materially increases the amount of area or space occupied by the facility on, over, above, along, under, across or within the right-of-way, without first filing an application with the Superintendent and obtaining a permit from the Village therefor, except as otherwise provided in this Article.


No permit shall be required for installation and maintenance of service connections to customers’ premises where there will be no disruption of the right-of-way.


(B)

Permit Application.  All applications for permits pursuant to this Article shall be filed on a form provided by the Village and shall be filed in such number of duplicate copies as the Village may designate.  The applicant may designate those portions of its application materials that it reasonably believes contain proprietary or confidential information as “proprietary” or “confidential” by clearly marking each page of such materials accordingly.


(C)

Minimum General Application Requirements.  The application shall be made by the utility or its duly authorized representative and shall contain, at a minimum, the following:

(1)
The utility’s name and address and telephone and telecopy numbers;

(2)
The applicant’s name and address, if different than the utility, its telephone, telecopy number, e-mail address, and its interest in the work;

(3)
The names, addresses and telephone and telecopy numbers and e-mail addresses of all professional consultants, if any, advising the applicant with respect to the application.

(4)
A general description of the proposed work and the purposes and intent of the facility and the uses to which the facility will be put.  The scope and detail of such description shall be appropriate to the nature and character of the work to be performed, with 

special emphasis on those matters likely to be affected or impacted by the work proposed;

(5)
Evidence that the utility has placed on file with the Village:

(a)
A written traffic control plan demonstrating the protective measures and devices that will be employed consistent with the Illinois Manual on Uniform Traffic Control Devices, to prevent injury or damage to persons or property and to minimize disruptions to efficient pedestrian and vehicular traffic; and

(b)
An emergency contingency plan which shall specify the nature of potential emergencies, including, without limitation, construction and hazardous materials emergencies, and the intended response by the applicant.  The intended response shall include notification to the Village and shall promote protection of the safety and convenience of the public.  Compliance with ICC regulations for emergency contingency plans constitutes compliance with this Section unless the Village finds that additional information or assurances are needed;

(6)
Drawings, plans and specifications showing the work proposed, including the certification of an engineer that such drawings, plans, and specifications comply with applicable codes, rules, and regulations;

(7)
Evidence of insurance as required in Section 33-4-8 of this Article;

(8)
Evidence of posting of the security fund as required in Section 33-4-10 of this Article;

(9)
Any request for a variance from one or more provisions of this Article (See Section 33-4-21); and

(10)
Such additional information as may be reasonably required by the Village.


(D)

Supplemental Application Requirements for Specific Types of Utilities.  In addition to the requirements of paragraph (C) of this Section, the permit application shall include the following items, as applicable to the specific utility that is the subject of the permit application:

(1)
In the case of the installation of a new electric power, communications, telecommunications, cable television service, video service or natural gas distribution system, evidence that any “Certificate of Public Convenience and Necessity” or other regulatory authorization that the applicant is required by law to obtain, or that the applicant has elected to obtain, has been issued by the ICC or other jurisdictional authority;

(2)
In the case of natural gas systems, state the proposed pipe size, design, construction class, and operating pressures;

(3)
In the case of water lines, indicate that all requirements of the Illinois Environmental Protection Agency, Division of Public Water Supplies, have been satisfied;

(4)
In the case of sewer line installations, indicate that the land and water pollution requirements of the Illinois Environmental Protection Agency, Division of Water Pollution Control have been satisfied; or

(5)
In the case of petroleum products pipelines, state the type or types of petroleum products, pipe size, maximum working pressure, and the design standard to be followed.


(E)

Applicant’s Duty to Update Information.  Throughout the entire permit application review period and the construction period authorized by the permit, any amendments to information contained in a permit application shall be submitted by the utility in writing to the Village within thirty (30) days after the change necessitating the amendment.


(F)

Application Fees.  Unless otherwise provided by franchise, license, or similar agreement, all applications for permits pursuant to this Article shall be accompanied by a base fee in the amount of $100.00.  Additional application review costs will vary with the complexity of the project and will be determined on a case by case basis and shall be paid prior to issuance of final permit.  No application fee is required to be paid by any electricity utility that is paying the municipal electricity infrastructure maintenance fee pursuant to the Electricity Infrastructure Maintenance Fee Act.


33-4-5
ACTION ON PERMIT APPLICATIONS.


(A)

Village Review of Permit Applications.  Completed permit applications, containing all required documentation, shall be examined by the Superintendent within a reasonable time after filing.  If the application does not conform to the requirements of all applicable ordinances, codes, laws, rules, and regulations, the Superintendent shall reject such application in writing, stating the reasons therefor.  If the Superintendent is satisfied that the proposed work conforms to the requirements of this Article and all applicable ordinances, codes, laws, rules, and regulations, the Superintendent shall issue a permit therefor as soon as practicable.  In all instances, it shall be the duty of the applicant to demonstrate, to the satisfaction of the Superintendent, that the construction proposed under the application shall be in full compliance with the requirements of this Article.


(B)

Additional Village Review of Applications of Telecommunications Retailers.

(1)
Pursuant to Section 4 of the Telephone Company Act, 220 ILCS 65/4, a telecommunications retailer shall notify the Village that it intends to commence work governed by this Article for facilities for the provision of telecommunications services.  Such notice shall consist of plans, specifications, and other documentation sufficient to demonstrate the purpose and intent of the facilities, and shall be provided by the telecommunications retailer to the Village not less than ten (10) days prior to the commencement of work requiring no excavation and not less than thirty (30) days prior to the commencement of work requiring excavation.  The Superintendent shall specify the portion of the right-of-way upon which the facility may be placed, used or constructed.

(2)
In the event that the Superintendent fails to provide such specification of location to the telecommunications retailer within either (a) ten (10) days after service of notice to the Village by 

the telecommunications retailer in the case of work not involving excavation for new construction or (b) twenty-five (25) days after service of notice by the telecommunications retailer in the case of work involving excavation for new construction, the telecommunications retailer may commence work without obtaining a permit under this Article.

(3)
Upon the provision of such specification by the Village, where a permit is required for work pursuant to Section 33-4-4 of this Article the telecommunications retailer shall submit to the Village an application for a permit and any and all plans, specifications and documentation available regarding the facility to be constructed.  Such application shall be subject to the requirements of paragraph (A) of this Section.


(C)

Additional Village Review of Applications of Holders of State Authorization Under the Cable and Video Competition Law of 2007.  Applications by a utility that is a holder of a State-issued authorization under the Cable and Video Competition Law of 2007 shall be deemed granted forty-five (45) days after submission to the Village, unless otherwise acted upon by the Village, provided the holder has complied with applicable Village codes, ordinances, and regulations.


33-4-6
EFFECT OF PERMIT.


(A)

Authority Granted; No Property Right or Other Interest Created.  A permit from the Village authorizes a permittee to undertake only certain activities in accordance with this Article on Village rights-of-way, and does not create a property right or grant authority to the permittee to impinge upon the rights of others who may have an interest in the public rights-of-way.


(B)

Duration.  No permit issued under this Article shall be valid for a period longer than six (6) months unless construction is actually begun within that period and is thereafter diligently pursued to completion.


(C)

Pre-Construction Meeting Required.  No construction shall begin pursuant to a permit issued under this Article prior to attendance by the permittee and all major contractors and subcontractors who will perform any work under the permit at a pre-construction meeting.  The pre-construction meeting shall be held at a date, time and place designated by the Village with such Village representatives in attendance as the Village deems necessary.  The meeting shall be for the purpose of reviewing the work under the permit, and reviewing special considerations necessary in the areas where work will occur, including, without limitation, presence or absence of other utility facilities in the area and their locations, procedures to avoid disruption of other utilities, use of rights-of-way by the public during construction, and access and egress by adjacent property owners.


(D)

Compliance With All Laws Required.  The issuance of a permit by the Village does not excuse the permittee from complying with other requirements of the Village and applicable statutes, laws, ordinances, rules, and regulations.


33-4-7
REVISED PERMIT DRAWINGS.  In the event that the actual locations of any facilities deviate in any material respect from the locations identified in the plans, drawings and specifications submitted with the permit application, the permittee shall submit a 

revised set of drawings or plans to the Village within ninety (90) days after the completion of the permitted work.  The revised drawings or plans shall specifically identify where the locations of the actual facilities deviate from the locations approved in the permit.  If any deviation from the permit also deviates from the requirements of this Article, it shall be treated as a request for variance in accordance with Section 33-4-21 of this Article.  If the Village denies the request for a variance, then the permittee shall either remove the facility from the right-of-way or modify the facility so that it conforms to the permit and submit revised drawings or plans therefor.


33-4-8
INSURANCE.


(A)

Required Coverages and Limits.  Unless otherwise provided by franchise, license, or similar agreement, each utility occupying right-of-way or constructing any facility in the right-of-way shall secure and maintain the following liability insurance policies insuring the utility as named insured and naming the Village, and its elected and appointed officers, officials, agents, and employees as additional insureds on the policies listed in paragraphs (1) and (2) below:

(1)
Commercial general liability insurance, including premises-operations, explosion, collapse, and underground hazard (commonly referred as “X”, “C”, and “U” coverages) and products-completed operations coverage with limits not less than:

(a)
Five Million Dollars ($5,000,000.00) for bodily injury or death to each person;

(b)
Five Million Dollars ($5,000,000.00) for property damage resulting from any one accident; and

(c)
Five Million Dollars ($5,000,000.00) for all other types of liability;

(2)
Automobile liability for owned, non-owned and hired vehicles with a combined single limit of One Million Dollars ($1,000,000.00) for personal injury and property damage for each accident;

(3)
Worker’s compensation with statutory limits; and

(4)
Employer’s liability insurance with limits of not less than One Million Dollars ($1,000,000.00) per employee and per accident.


If the utility is not providing such insurance to protect the contractors and subcontractors performing the work, then such contractors and subcontractors shall comply with this Section.


(B)

Excess or Umbrella Policies.  The coverages required by this Section may be in any combination of primary, excess, and umbrella policies.  Any excess or umbrella policy must provide excess coverage over underlying insurance on a following-form basis such that when any loss covered by the primary policy exceeds the limits under the primary policy, the excess or umbrella policy becomes effective to cover such loss.


(C)

Copies Required.  The utility shall provide copies of any of the policies required by this Section to the Village within ten (10) days following receipt of a written request therefor from the Village.


(D)

Maintenance and Renewal of Required Coverages.  The insurance policies required by this Section shall contain the following endorsement:

“It is hereby understood and agreed that this policy may not be canceled nor the intention not to renew be stated until thirty (30) days after receipt by the Village, by registered mail or certified mail, return receipt requested, of a written notice addressed to the Village Administrator of such intent to cancel or not to renew.”


Within ten (10) days after receipt by the Village of said notice, and in no event later than ten (10) days prior to said cancellation, the utility shall obtain and furnish to the Village evidence of replacement insurance policies meeting the requirements of this Section.


(E)

Self-Insurance.  A utility may self-insure all or a portion of the insurance coverage and limit requirements required by paragraph (A) of this Section.  A utility that self-insures is not required, to the extent of such self-insurance, to comply with the requirement for the naming of additional insureds under paragraph (A), or the requirements of paragraphs (B), (C) and (D) of this Section.  A utility that elects to self-insure shall provide to the Village evidence sufficient to demonstrate its financial ability to self-insure the insurance coverage and limit requirements required under paragraph (A) of this Section, such as evidence that the utility is a “private self insurer” under the Workers Compensation Act.


(F)

Effect of Insurance and Self-Insurance on Utility’s Liability.  The legal liability of the utility to the Village and any person for any of the matters that are the subject of the insurance policies or self-insurance required by this Section shall not be limited by such insurance policies or self-insurance or by the recovery of any amounts thereunder.


(G)

Insurance Companies.  All insurance provided pursuant to this Section shall be effected under valid and enforceable policies, issued by insurers legally able to conduct business with the licensee in the State of Illinois.  All insurance carriers and surplus line carriers shall be rates “A-“ or better and of a class size “X” or higher by A.M. Best Company.


33-4-9
INDEMNIFICATION.  By occupying or constructing facilities in the right-of-way, a utility shall be deemed to agree to defend, indemnify and hold the Village and its elected and appointed officials and officers, employees, agents and representatives harmless from and against any and all injuries, claims, demands, judgments, damages, losses and expenses, including reasonable attorney’s fees and costs of suit or defense, arising out of, resulting from or alleged to arise out of or result from the negligent, careless or wrongful acts, omissions, failures to act or misconduct of the utility or its affiliates, officers, employees, agents, contractors or subcontractors in the construction of facilities or occupancy of the rights-of-way, and in providing or offering service over the facilities, whether such acts or omissions are authorized, allowed or prohibited by this Article or by a franchise, license, or similar agreement; provided, however, that the utility’s indemnity obligations hereunder shall not apply to any injuries, claims, demands, judgments, damages, losses or expenses arising out of or resulting from the negligence, misconduct or breach of this Article by the Village, its officials, officers, employees, agents or representatives.


33-4-10
SECURITY.


(A)

Purpose.  The permittee shall establish a Security Fund in a form and in an amount as set forth in this Section.  The Security Fund shall be continuously maintained in accordance with this Section at the permittee’s sole cost and expense until the completion of the work authorized under the permit.  The Security Fund shall serve a security for:

(1)
The faithful performance by the permittee of all the requirements of this Article;

(2)
Any expenditure, damage, or loss incurred by the Village occasioned by the permittee’s failure to comply with any codes, rules, regulations, orders, permits and other directives of the Village issued pursuant to this Article; and

(3)
The payment by permittee of all liens and all damages, claims, costs, or expenses that the Village may pay or incur by reason of any action or non-performance by permittee in violation of this Article including, without limitation, any damage to public property or restoration work the permittee is required by this Article to perform that the Village must perform itself or have completed as a consequence solely of the permittee’s failure to perform or complete, and all other payments due the Village from the permittee pursuant to this Article or any other applicable law.


(B)

Form.  The permittee shall provide the Security Fund to the Village in the form, at the permittee’s election, of cash, a surety bond in a form acceptable to the Village, or an unconditional letter of credit in a form acceptable to the Village.  Any surety bond or letter of credit provided pursuant to this paragraph shall, at a minimum:

(1)
Provide that it will not be canceled without prior notice to the Village and the permittee;

(2)
Not require the consent of the permittee prior to the collection by the Village of any amounts covered by it; and

(3)
Shall provide a location convenient to the Village and within the State of Illinois at which it can be drawn.


(C)

Amount.  The dollar amount of the Security Fund shall be sufficient to provide for the reasonably estimated cost to restore the right-of-way to at least as good a condition as that existing prior to the construction under the permit, as determined by the Superintendent, and may also include reasonable, directly related costs that the Village estimates are likely to be incurred if the permittee fails to perform such restoration.  Where the construction of facilities proposed under the permit will be performed in phases in multiple locations in the Village, with each phase consisting of construction of facilities in one location or related group of locations, and where construction in another phase will not be undertaken prior to substantial completion of restoration in the previous phase or phases, the Superintendent may, in the exercise of sound discretion, allow the permittee to post a single amount of security which shall be applicable to each phase of the construction under the permit.  The amount of the Security Fund for phased construction shall be equal to the greatest amount that would have been required under the provisions of this paragraph (C) for any single phase.


(D)

Withdrawals.  The Village, upon fourteen (14) days’ advance written notice clearly stating the reason for, and its intention to exercise withdrawal rights under this paragraph, may withdraw an amount from the Security Fund, provided that the permittee has not reimbursed the Village for such amount within the fourteen (14) day notice period.  Withdrawals may be made if the permittee:

(1)
Fails to make any payment required to be made by the permittee hereunder;

(2)
Fails to pay any liens relating to the facilities that are due and unpaid;

(3)
Fails to reimburse the Village for any damages, claims, costs or expenses which the Village has been compelled to pay or incur by reason of any action or non-performance by the permittee; or

(4)
Fails to comply with any provision of this Article that the Village determines can be remedied by an expenditure of an amount in the Security Fund.


(E)

Replenishment.  Within fourteen (14) days after receipt of written notice from the Village that any amount has been withdrawn from the Security Fund, the permittee shall restore the Security Fund to the amount specified in paragraph (C) of this Section.


(F)

Interest.  The permittee may request that any and all interest accrued on the amount in the Security Fund be returned to the permittee by the Village, upon written request for said withdrawal to the Village, provided that any such withdrawal does not reduce the Security Fund below the minimum balance required in paragraph (C) of this Section.


(G)

Closing and Return of Security Fund.  Upon completion of the work authorized under the permit, the permittee shall be entitled to the return of the Security Fund, or such portion thereof as remains on deposit, within a reasonable time after account is taken for all offsets necessary to compensate the Village for failure by the permittee to comply with any provisions of this Article or other applicable law.  In the event of any revocation of the permit, the Security Fund, and any and all accrued interest therein, shall become the property of the Village to the extent necessary to cover any reasonable costs, loss or damage incurred by the Village as a result of said revocation, provided that any amounts in excess of said costs, loss or damage shall be refunded to the permittee.


(H)

Rights Not Limited.  The rights reserved to the Village with respect to the Security Fund are in addition to all other rights of the Village, whether reserved by this Article or otherwise authorized by law, and no action, proceeding or exercise of right with respect to said Security Fund shall affect any other right the Village may have.  Notwithstanding the foregoing, the Village shall not be entitled to a double monetary recovery with respect to any of its rights which may be infringed or otherwise violated.


33-4-11
PERMIT SUSPENSION AND REVOCATION.


(A)

Village Right to Revoke Permit.  The Village may revoke or suspend a permit issued pursuant to this Article for one or more of the following reasons:

(1)
Fraudulent, false, misrepresenting, or materially incomplete statements in the permit application;

(2)
Noncompliance with this Article;

(3)
Permittee’s physical presence or presence of permittee’s facilities on, over, above, along, upon, under, across, or within the rights-of-way presents a direct or imminent threat to the public health, safety, or welfare; or

(4)
Permittee’s failure to construct the facilities substantially in accordance with the permit and approved plans.


(B)

Notice of Revocation or Suspension.  The Village shall send written notice of its intent to revoke or suspend a permit issued pursuant to this Article stating the reason or reasons for the revocation or suspension and the alternatives available to permittee under this Section 33-4-11.


(C)

Permittee Alternatives Upon Receipt of Notice of Revocation or Suspension.  Upon receipt of a written notice of revocation or suspension from the Village, the permittee shall have the following options:

(1)
Immediately provide the Village with evidence that no cause exists for the revocation or suspension;

(2)
Immediately correct, to the satisfaction of the Village, the deficiencies stated in the written notice, providing written proof of such correction to the Village within five (5) working days after receipt of the written notice of revocation; or

(3)
Immediately remove the facilities located on, over, above, along, upon, under, across, or within the rights-of-way and restore the rights-of-way to the satisfaction of the Village providing written proof of such removal to the Village within ten (10) days after receipt of the written notice of revocation.

The Village may, in its discretion, for good cause shown, extend the time periods provided in this paragraph.


(D)

Stop Work Order.  In addition to the issuance of a notice of revocation or suspension, the Village may issue a stop work order immediately upon discovery of any of the reasons for revocation set forth within paragraph (A) of this Section.


(E)

Failure or Refusal of the Permittee to Comply.  If the permittee fails to comply with the provisions of paragraph (C) of this Section, the Village or its designee may, at the option of the Village:

(1)
correct the deficiencies;

(2)
upon not less than twenty (20) days notice to the permittee, remove the subject facilities or equipment; or

(3)
after not less than thirty (30) days notice to the permittee of failure to cure the noncompliance, deem them abandoned and property of the Village.  The permittee shall be liable in all events to the Village for all costs of removal.


33-4-12
CHANGE OF OWNERSHIP OR OWNER’S IDENTITY OR LEGAL STATUS.


(A)

Notification of Change.  A utility shall notify the Village no less than thirty (30) days prior to the transfer of ownership of any facility in the right-of-way or change in identity of the utility.  The new owner of the utility or the facility shall have all the obligations and privileges enjoyed by the former owner under the permit, if any, and all applicable laws, ordinances, rules and regulations, including this Article, with respect to the work and facilities in the right-of-way.


(B)

Amended Permit.  A new owner shall request that any current permit be amended to show current ownership.  If the new owner fails to have a new or amended permit issued in its name, the new owner shall be presumed to have accepted, and agreed to be bound by, the terms and conditions of the permit if the new owner uses the facility or allows it to remain on the Village’s right-of-way.


(C)

Insurance and Bonding.  All required insurance coverage or bonding must be changed to reflect the name of the new owner upon transfer.


33-4-13
GENERAL CONSTRUCTION STANDARDS.


(A)

Standards and Principles.  All construction in the right-of-way shall be consistent with applicable ordinances, codes, laws, rules and regulations, and commonly recognized and accepted traffic control and construction principles, sound engineering judgment and, where applicable, the principles and standards set forth in the following IDOT publications, as amended from time to time:

(1)
Standard Specifications for Road and Bridge Construction;

(2)
Supplemental Specifications and Recurring Special Provisions;

(3)
Highway Design Manual;

(4)
Highway Standards Manual;

(5)
Standard Specifications for Traffic Control Items;

(6)
Illinois Manual on Uniform Traffic Control Devices (92 Ill. Adm. Code § 545);

(7)
Flagger’s Handbook; and

(8)
Work Site Protection Manual for Daylight Maintenance Operations.


(B)

Interpretation of Municipal Standards and Principles.  If a discrepancy exists between or among differing principles and standards required by this Article, the Superintendent shall determine, in the exercise of sound engineering judgment, which principles apply and such decision shall be final.  If requested, the Superintendent shall state which standard or principle will apply to the construction, maintenance, or operation of a facility in the future.


33-4-14
TRAFFIC CONTROL.


(A)

Minimum Requirements.  The Village’s minimum requirements for traffic protection are contained in IDOT’s Illinois Manual on Uniform Traffic Control Devices and this Code.


(B)

Warning Signs, Protective Devices, and Flaggers.  The utility is responsible for providing and installing warning signs, protective devices and flaggers, when necessary, meeting all applicable federal, state, and local requirements for protection of the public and the utility’s workers when performing any work on the public rights-of-way.


(C)

Interference with Traffic.  All work shall be phased so that there is minimum interference with pedestrian and vehicular traffic.


(D)

Notice When Access is Blocked.  At least forty-eight (48) hours prior to beginning work that will partially or completely block access to any residence, business or institution, the utility shall notify the resident, business or institution of the approximate beginning time and duration of such work; provided, however, that in cases involving emergency repairs pursuant to Section 33-4-20 of this Article, the utility shall provide such notice as is practicable under the circumstances.


(E)

Compliance.  The utility shall take immediate action to correct any deficiencies in traffic protection requirements that are brought to the utility’s attention by the Village.


33-4-15
LOCATION OF FACILITIES.


(A)

General Requirements.  In addition to location requirements applicable to specific types of utility facilities, all utility facilities, regardless of type, shall be subject to the general location requirements of this paragraph.

(1)
No Interference with Village Facilities.  No utility facilities shall be placed in any location if the Superintendent determines that the proposed location will require the relocation or displacement of any of the Village’s utility facilities or will otherwise interfere with the operation or maintenance of any of the Village’s utility facilities.

(2)
Minimum Interference and Impact.  The proposed location shall cause only the minimum possible interference with the use of the right-of-way and shall cause only the minimum possible impact upon, and interference with the rights and reasonable convenience of property owners who adjoin said right-of-way.

(3)
No Interference with Travel.  No utility facility shall be placed in any location that interferes with the usual travel on such right-of-way.

(4)
No Limitations on Visibility.  No utility facility shall be placed in any location so as to limit visibility of or by users of the right-of-way.

(5)
Size of Utility Facilities.  The proposed installation shall use the smallest suitable vaults, boxes, equipment enclosures, power pedestals, and/or cabinets then in use by the facility owner, regardless of location, for the particular application.


(B)

Parallel Facilities Located Within Highways.

(1)
Overhead Parallel Facilities.  An overhead parallel facility may be located within the right-of-way lines of a highway only if:

(a)
Lines are located as near as practicable to the right-of-way line and as nearly parallel to the right-of-way line as reasonable pole alignment will permit;

(b)
Where pavement is curbed, poles are as remote as practicable from the curb with a minimum distance of two (2) feet (0.6m) behind the face of the curb, where available;

(c)
Where pavement is uncurbed, poles are as remote from pavement edge as practicable with minimum distance of four (4) feet (1.2m) outside the outer shoulder line of the roadway and are not within the clear zone;

(d)
No pole is located in the ditch line of a highway; and

(e)
Any ground-mounted appurtenance is located within one (1) foot (0.3m) of the right-of-way line or as near as possible to the right-of-way line.

(2)
Underground Parallel Facilities.  An underground parallel facility may be located within the right-of-way lines of a highway only if:

(a)
The facility is located as near the right-of-way line as practicable and not more than eight (8) feet (2.4m) from and parallel to the right-of-way line;

(b)
A new facility may be located under the paved portion of a highway only if other locations are impracticable or inconsistent with sound engineering judgment (e.g., a new 

cable may be installed in existing conduit without disrupting the pavement); and

(c)
In the case of an underground power or communications line, the facility shall be located as near the right-of-way line as practicable and not more than five (5) feet (1.5m) from the right-of-way line and any above-grounded appurtenance shall be located within one (1) foot (0.3m) of the right-of-way line or as near as practicable.


(C)

Facilities Crossing Highways.

(1)
No Future Disruption.  The construction and design of crossing facilities installed between the ditch lines or curb lines of Village highways may require the incorporation of materials and protections (such as encasement or additional cover) to avoid settlement or future repairs to the roadbed resulting from the installation of such crossing facilities.

(2)
Cattle Passes, Culverts, or Drainage Facilities.  Crossing facilities shall not be located in cattle passes, culverts, or drainage facilities.

(3)
90 Degree Crossing Required.  Crossing facilities shall cross at or as near to a ninety (90) degree angle to the centerline as practicable.

(4)
Overhead Power or Communication Facility.  An overhead power or communication facility may cross a highway only if:

(a)
It has a minimum vertical line clearance as required by ICC’s rules entitled, “Construction of Electric Power and Communication Lines” (83 Ill. Adm. Code 305);

(b)
Poles are located within one (1) foot (0.3m) of the right-of-way line of the highway and outside of the clear zone; and

(c)
Overhead crossings at major intersections are avoided.

(5)
Underground Power or Communication Facility.  An underground power or communication facility may cross a highway only if:

(a)
The design materials and construction methods will provide maximum maintenance-free service life; and

(b)
Capacity for the utility’s foreseeable future expansion needs is provided in the initial installation.

(6)
Markers.  The Village may require the utility to provide a marker at each right-of-way line where an underground facility other than a power or communication facility crosses a highway.  Each marker shall identify the type of facility, the utility, and an emergency phone number.  Markers may also be eliminated as provided in current Federal regulations.  (49 C.F.R. §192.707 (1989)).


(D)

Facilities to be Located Within Particular Rights-of-Way.  The Village may require that facilities be located within particular rights-of-way that are not highways, rather than within particular highways.


(E)

Freestanding Facilities.

(1)
The Village may restrict the location and size of any freestanding facility located within a right-of-way.

(2)
The Village may require any freestanding facility located within a right-of-way to be screened from view.


(F)

Facilities Installed Above Ground.  Above ground facilities may be installed only if:

(1)
No other existing facilities in the area are located underground;

(2)
New underground installation is not technically feasible; and

(3)
The proposed installation will be made at a location, and will employ suitable design and materials, to provide the greatest protection of aesthetic qualities of the area being traversed without adversely affecting safety.  Suitable designs include, but are not limited to, self-supporting armless, single-pole construction with vertical configuration of conductors and cable.  Existing utility poles and light standards shall be used wherever practicable; the installation of additional utility poles is strongly discouraged.


(G)

Facility Attachments to Bridges or Roadway Structures.

(1)
Facilities may be installed as attachments to bridges or roadway structures only where the utility has demonstrated that all other means of accommodating the facility are not practicable.  Other means shall include, but are not limited to, underground, underwater, independent poles, cable supports and tower supports, all of which are completely separated from the bridge or roadway structure.  Facilities transmitting commodities that are volatile, flammable, corrosive, or energized, especially those under significant pressure or potential, present high degrees of risk and such installations are not permitted.

(2)
A utility shall include in its request to accommodate a facility installation on a bridge or roadway structure supporting data demonstrating the impracticability of alternate routing.  Approval or disapproval of an application for facility attachment to a bridge or roadway structure will be based upon the following considerations:

(a)
The type, volume, pressure or voltage of the commodity to be transmitted and an evaluation of the resulting risk to persons and property in the event of damage to or failure of the facility;

(b)
The type, length, value, and relative importance of the highway structure in the transportation system;

(c)
The alternative routings available to the utility and their comparative practicability;

(d)
The proposed method of attachment;

(e)
The ability of the structure to bear the increased load of the proposed facility;

(f)
The degree of interference with bridge maintenance and painting;

(g)
The effect on the visual quality of the structure; and

(h)
The public benefit expected from the utility service as compared to the risk involved.


(H)

Appearance Standards.

(1)
The Village may prohibit the installation of facilities in particular locations in order to preserve visual quality.

(2)
A facility may be constructed only if its construction does not require extensive removal or alteration of trees or terrain features visible to the right-of-way user or to adjacent residents and property owners, and if it does not impair the aesthetic quality of the lands being traversed.


33-4-16
CONSTRUCTION METHODS AND MATERIALS.


(A)

Standards and Requirements for Particular Types of Construction Methods.

(1)
Boring or Jacking.

(a)
Pits and Shoring.  Boring and jacking under rights-of-way shall be accomplished from pits located at a minimum distance specified by the Superintendent from the edge of the pavement.  Pits for boring or jacking shall be excavated no more than forty-eight (48) hours in advance of boring or jacking operations and backfilled within forty-eight (48) hours after boring or jacking operations are completed.  While pits are open, they shall be clearly marked and protected by barricades.  Shoring shall be designed, erected, supported, braced, and maintained so that it will safely support all vertical and lateral loads that may be imposed upon it during the boring or jacking operation.

(b)
Wet Boring or Jetting.  Wet boring or jetting shall not be permitted under the roadway.

(c)
Borings With Diameters Greater than Six (6) Inches.  Borings over six (6) inches (0.15m) in diameter shall be accomplished with an auger and following pipe, and the diameter of the auger shall not exceed the outside diameter of the following pipe by more than one (1) inch (25mm).

(d)
Borings with Diameters Six (6) Inches or Less.  Borings of six (6) inches or less in diameter may be accomplished by either jacking, guided with auger, or auger and following pipe method.

(e)
Tree Preservation.  Any facility located within the drip line of any tree designed by the Village to be preserved shall be bored under or around the root system.

(2)
Trenching.  Trenching for facility installation, repair, or maintenance on rights-of-way shall be done in accord with the 

applicable portions of Section 603 IDOT’s “Standard Specifications for Road and Bridge Construction”.

(a)
Length.  The length of open trench shall be kept to the practicable minimum consistent with requirements for pipe-line testing.  Only one-half of any intersection may have an open trench at any time unless special permission is obtained from the Superintendent.

(b)
Open Trench and Excavated Material.  Open trench and wind rowed excavated material shall be protected as required by Chapter 6 of the Illinois Manual on Uniform Traffic Control Devices.  Where practicable, the excavated material shall be deposited between the roadway and the trench as added protection.  Excavated material shall not be allowed to remain on the paved portion of the roadway.  Where right-of-way width does not allow for wind rowing excavated material off the paved portion of the roadway, excavated material shall be hauled to an off-road location.

(c)
Drip Line of Trees.  The utility shall not trench within the drip line of any tree designated by the Village to be preserved.
(3)
Backfilling.

(a)
Any pit, trench, or excavation created during the installation of facilities shall be backfilled for its full width, depth, and length using methods and materials in accordance with IDOT’s “Standard Specifications for Road and Bridge Construction”.  When excavated material is hauled away or is unsuitable for backfill, suitable granular backfill shall be used.

(b)
For a period of three (3) years from the date construction of a facility is completed, the utility shall be responsible to remove and restore any backfilled area that has settled due to construction of the facility.  If so ordered by the Superintendent, the utility, at its expense, shall remove any pavement and backfill material to the top of the installed facility, place and properly compact new backfill material, and restore new pavement, sidewalk, curbs, and driveways to the proper grades, as determined by the Superintendent.

(4)
Pavement Cuts.  Pavement cuts for facility installation or repair shall be discouraged but may be permitted only upon location specific approval by the Superintendent and only if that portion of the roadway is closed to traffic.

(a)
Any excavation under pavements shall be backfilled and compacted as soon as practicable with granular material of CA-6 or CA-10 gradation, as designated by the Superintendent.

(b)
Restoration of pavement, in kind, shall be accomplished as soon as practicable, and temporary repair with bituminous 

mixture shall be provided immediately.  Any subsequent failure of either the temporary repair or the restoration shall be rebuilt upon notification by the Village.

(c)
All saw cuts shall be full depth.

(d)
For all rights-of-way which have been reconstructed with a concrete surface/base in the last seven (7) years, or resurfaced in the last three (3) years, permits shall not be issued unless such work is determined to be an emergency repair or other work considered necessary and unforeseen before the time of the reconstruction or unless a pavement cut is necessary for a J.U.L.I.E. locate.

(5)
Encasement.

(a)
Casing pipe shall be designed to withstand the load of the highway and any other superimposed loads.  The casing shall be continuous either by one-piece fabrication or by welding or jointed installation approved by the Village.

(b)
The venting, if any, of any encasement shall extend within one (1) foot (0.3m) of the right-of-way line.  No above-ground vent pipes shall be located in the area established as clear zone for that particular section of the highway.

(c)
In the case of water main or service crossing, encasement shall be furnished between bore pits unless continuous pipe or Village approved jointed pipe is used under the roadway.  Casing may be omitted only if pipe is installed prior to highway construction and carrier pipe is continuous or mechanical joints are of a type approved by the Village.  Bell and spigot type pipe shall be encased regardless of installation method.

(d)
In the case of gas pipelines of 60 psig or less, encasement may be eliminated.

(e)
In the case of gas pipelines or petroleum products pipelines with installations of more than 60 psig, encasement may be eliminated only if:

(i)
extra heavy pipe is used that precludes future maintenance or repair and

(ii)
cathodic protection of the pipe is provided;

(f)
If encasement is eliminated for a gas or petroleum products pipeline, the facility shall be located so as to provide that construction does not disrupt the right-of-way.

(6)
Minimum Cover of Underground Facilities.  Cover shall be provided and maintained at least in the amount specified in the following table for minimum cover for the type of facility:

Type of Facility
Minimum Cover



Electric Lines
30 inches (0.8m)



Communication, Cable or Video

Service Lines
18 to 24 inches (0.6m, as


Determined by Village)


Gas or Petroleum Products
30 inches (0.8m)



Water Line
Sufficient Cover to Provide Freeze Protection

Sanitary Sewer, Storm Sewer,

Or Drainage Line
Sufficient Cover to Provide Freeze Protection


(B)

Standards and Requirements for Particular Types of Facilities.

(1)
Electric Power or Communication Lines.

(a)
Code Compliance.  Electric power or communications facilities within Village rights-of-way shall be constructed, operated, and maintained in conformity with the provisions of 83 Ill. Adm. Code 305 (formerly General Order 160 of the Illinois Commerce Commission) entitled “Rules for Construction of Electric Power and Communications Lines”, and the National Electrical Safety Code.

(b)
Overhead Facilities.  Overhead power or communication facilities shall use single pole construction and, where practicable, joint use of poles shall be used.  Utilities shall make every reasonable effort to design the installation so guys and braces will not be needed.  Variances may be allowed if there is no feasible alternative and if guy wires are equipped with guy guards for maximum visibility.

(c)
Underground Facilities.

(i)
Cable may be installed by trenching or plowing, provided that special consideration is given to boring in order to minimize damage when crossing improved entrances and side roads.

(ii)
If a crossing is installed by boring or jacking, encasement shall be provided between jacking or bore pits.  Encasement may be eliminated only if:

a.
the crossing is installed by the use of “moles”, “whip augers”, or other approved method which compress the earth to make the opening for cable installation or

b.
the installation is by the open trench method which is only permitted prior to roadway construction.

(iii)
Cable shall be grounded in accordance with the National Electrical Safety Code.

(iv)
Burial of Drops.  All temporary service drops placed between November 1 of the prior year and March 15 of the current year, also known as snowdrops, shall be buried by May 31 of the current year, weather permitting, unless otherwise permitted by the Village.  Weather permitting, utilities shall bury all temporary drops, excluding snowdrops, within ten (10) business days after placement.

(2)
Underground Facilities Other Than Electric Power or Communication Lines.  Underground facilities other than electric power or communication lines may be installed by:

(a)
The use of “moles”, “whip augers”, or other approved methods which compress the earth to move the opening for the pipe;

(b)
jacking or boring with vented encasement provided between the ditch lines or toes of slopes of the highway;

(c)
open trench with vented encasement between ultimate ditch lines or toes of slopes, but only if prior to roadway construction; or

(d)
tunneling with vented encasement, but only if installation is not possible by other means.

(3)
Gas Transmission, Distribution and Service.  Gas pipelines within rights-of-way shall be constructed, maintained, and operated in a Village approved manner and in conformance with the Federal Code of the Office of Pipeline Safety Operations, Department of Transportation, Part 192 – Transportation of Natural and Other Gas by Pipeline: Minimum Federal Safety Standards (49 CFR §192), IDOT’s “Standard Specifications for Road and Bridge Construction”, and all other applicable laws, rules, and regulations.

(4)
Petroleum Products Pipelines.  Petroleum products pipelines within rights-of-way shall conform to the applicable sections of ANSI Standard Code for Pressure Piping.  (Liquid Petroleum Transportation Piping Systems ANSI-B 31.4).

(5)
Waterlines, Sanitary Sewer Lines, Storm Water Sewer Lines or Drainage Lines.  Water lines, sanitary sewer lines, storm sewer lines, and drainage lines within rights-of-way shall meet or exceed the recommendations of the current “Standard Specifications for Water and Sewer Main Construction in Illinois”.

(6)
Ground Mounted Appurtenances.  Ground mounted appurtenances to overhead or underground facilities, when permitted within a right-of-way, shall be provided with a vegetation-free area extending one (1) foot (305mm) in width beyond the appurtenance in all directions.  The vegetation-free area may be provided by an extension of the mounting pad, or by heavy duty plastic or similar material approved by the Superintendent, shrubbery surrounding the appurtenance may be 

used in place of vegetation-free area.  The housing for ground-mounted appurtenances shall be painted a neutral color to blend with the surroundings.


(C)

Materials.

(1)
General Standards.  The materials used in constructing facilities within rights-of-way shall be those meeting the accepted standards of the appropriate industry, the applicable portions of IDOT’s “Standard Specifications for Road and Bridge Construction”, the requirements of the Illinois Commerce Commission, or the standards established by other official regulatory agencies for the appropriate industry.

(2)
Material Storage on Right-of-Way.  No material shall be stored on the right-of-way without the prior written approval of the Superintendent.  When such storage is permitted, all pipe, conduit, wire, poles, cross arms, or other materials shall be distributed along the right-of-way prior to and during installation in a manner to minimize hazards to the public or an obstacle to right-of-way maintenance or damage to the right-of-way and other property.  If material is to be stored on right-of-way, prior approval must be obtained from the Village.

(3)
Hazardous Materials.  The plans submitted by the utility to the Village shall identify any hazardous materials that may be involved in the construction of the new facilities or removal of any existing facilities.


(D)

Operational Restrictions.

(1)
Construction operations on rights-of-way may, at the discretion of the Village, be required to be discontinued when such operations would create hazards to traffic or the public health, safety, and welfare.  Such operations may also be required to be discontinued or restricted when conditions are such that construction would result in extensive damage to the right-of-way or other property.

(2)
These restrictions may be waived by the Superintendent when emergency work is required to restore vital utility services.

(3)
Unless otherwise permitted by the Village, the hours of construction are from 6:00 A.M. to 6:00 P.M.

(E)

Location of Existing Facilities.  Any utility proposing to construct facilities in the Village shall contact J.U.L.I.E. and ascertain the presence and location of existing above-ground and underground facilities within the rights-of-way to be occupied by its proposed facilities.  The Village will make its permit record available to a utility for the purpose of identifying possible facilities.  When notified of an excavation or when requested by the Village or by J.U.L.I.E., a utility shall locate and physically mark its underground facilities within forty-eight (48) hours, excluding weekends and holidays, in accordance with the Illinois Underground Facilities Damage Prevention Act (220 ILCS 50/1 et seq.)


33-4-17
VEGETATION CONTROL.


(A)

Electric Utilities - Compliance with State Laws and Regulations.  An electric utility shall conduct all tree-trimming and vegetation control activities in the right-of-

way in accordance with applicable Illinois laws and regulations, and additionally, with such local franchise or other agreement with the Village as permitted by law.


(B)

Other Utilities - Tree Trimming Permit Required.  Tree trimming that is done by any other utility with facilities in the right-of-way and that is not performed pursuant to applicable Illinois laws and regulations specifically governing the same, shall not be considered a normal maintenance operation, but shall require the application for, and the issuance of, a permit, in addition to any other permit required under this Article.

(1)
Application for Tree Trimming Permit.  Applications for tree trimming permits shall include assurance that the work will be accomplished by competent workers with supervision who are experienced in accepted tree pruning practices.  Tree trimming permits shall designate an expiration date in the interest of assuring that the work will be expeditiously accomplished.

(2)
Damage to Trees.  Poor pruning practices resulting in damaged or misshapen trees will not be tolerated and shall be grounds for cancellation of the tree trimming permit and for assessment of damages.  The Village will require compensation for trees extensively damaged and for trees removed without authorization.  The formula developed by the International Society of Arboriculture will be used as a basis for determining the compensation for damaged trees or unauthorized removal of trees.  The Village may require the removal and replacement of trees if trimming or radical pruning would leave them in an unacceptable condition.


(C)

Specimen Trees or Trees of Special Significance.  The Village may require that special measures be taken to preserve specimen trees or trees of special significance.  The required measures may consist of higher poles, side arm extensions, covered wire or other means.


(D)

Chemical Use.
(1)
Except as provided in the following paragraph, no utility shall spray, inject or pour any chemicals on or near any trees, shrubs or vegetation in the Village for any purpose, including the control of growth, insects or disease.

(2)
Spraying of any type of brush-killing chemicals will not be permitted on rights-of-way unless the utility demonstrates to the satisfaction of the Superintendent that such spraying is the only practicable method of vegetation control.


33-4-18
REMOVAL, RELOCATION, OR MODIFICATION OF UTILITY FACILITIES.


(A)

Notice.  Within ninety (90) days following written notice from the Village, a utility shall, at its own expense, protect, support, temporarily or permanently disconnect, remove, relocate, change or alter the position of any utility facilities within the rights-of-way whenever the corporate authorities have determined that such removal, relocation, change or alteration, is reasonably necessary for the construction, repair, maintenance, or installation of any Village improvement in or upon, or the operations of the Village in or upon, the rights-of-way.


(B)

Removal of Unauthorized Facilities.  Within thirty (30) days following written notice from the Village, any utility that owns, controls, or maintains any unauthorized facility or related appurtenances within the rights-of-way shall, at its own expense, remove all or any part of such facilities or appurtenances from the rights-of-way.  A facility is unauthorized and subject to removal in the following circumstances:

(1)
Upon expiration or termination of the permittee’s license or franchise, unless otherwise permitted by applicable law;

(2)
If the facility was constructed or installed without the prior grant of a license or franchise, if required;

(3)
If the facility was constructed or installed without prior issuance of a required permit in violation of this Article; or

(4)
If the facility was constructed or installed at a location not permitted by the permittee’s license or franchise.


(C)

Emergency Removal or Relocation of Facilities.  The Village retains the right and privilege to cut or move any facilities located within the rights-of-way of the Village, as the Village may determine to be necessary, appropriate or useful in response to any public health or safety emergency.  If circumstances permit, the Municipality shall attempt to notify the utility, if known, prior to cutting or removing a facility and shall notify the utility, if known, after cutting or removing a facility.


(D)

Abandonment of Facilities.  Upon abandonment of a facility within the rights-of-way of the Village, the utility shall notify the Village within ninety (90) days.  Following receipt of such notice the Village may direct the utility to remove all or any portion of the facility if the Superintendent determines that such removal will be in the best interest of the public health, safety and welfare.  In the event that the Village does not direct the utility that abandoned the facility to remove it, by giving notice of abandonment to the Village, the abandoning utility shall be deemed to consent to the alteration or removal of all or any portion of the facility by another utility or person.


33-4-19
CLEANUP AND RESTORATION.  The utility shall remove all excess material and restore all turf and terrain and other property within ten (10) days after any portion of the rights-of-way are disturbed, damaged or destroyed due to construction or maintenance by the utility, all to the satisfaction of the Village.  This includes restoration of entrances and side roads.  Restoration of roadway surfaces shall be made using materials and methods approved by the Superintendent.  Such cleanup and repair may be required to consist of backfilling, regrading, reseeding, resodding, or any other requirement to restore the right-of-way to a condition substantially equivalent to that which existed prior to the commencement of the project.  The time period provided in this Section may be extended by the Superintendent for good cause shown.


33-4-20
MAINTENANCE AND EMERGENCY MAINTENANCE.


(A)

General.  Facilities on, over, above, along, upon, under, across, or within rights-of-way are to be maintained by or for the utility in a manner satisfactory to the Village and at the utility’s expense.


(B)

Emergency Maintenance Procedures.  Emergencies may justify noncompliance with normal procedures for securing a permit:

(1)
If an emergency creates a hazard on the traveled portion of the right-of-way, the utility shall take immediate steps to provide all necessary protection for traffic on the highway or the public on the right-of-way including the use of signs, lights, barricades or flags.  If a hazard does not exist on the traveled way, but the nature of the emergency is such as to require the parking on the shoulder of equipment required in repair operations, adequate signs and lights shall be provided.  Parking on the shoulder in such an emergency will only be permitted when no other means of access to the facility is available.

(2)
In an emergency, the utility shall, as soon as possible, notify the Superintendent or his or her duly authorized agent of the emergency, informing him or her as to what steps have been taken for protection of the traveling public and what will be required to make the necessary repairs.  If the nature of the emergency is such as to interfere with the free movement of traffic, the Village police shall be notified immediately.

(3)
In an emergency, the utility shall use all means at hand to complete repairs as rapidly as practicable and with the least inconvenience to the traveling public.


(C)

Emergency Repairs.  The utility must file in writing with the Village a description of the repairs undertaken in the right-of-way within forty-eight (48) hours after an emergency repair.


33-4-21
VARIANCES.


(A)

Request for Variance.  A utility requesting a variance from one or more of the provisions of this Article must do so in writing to the Superintendent as a part of the permit application.  The request shall identify each provision of this Article from which a variance is requested and the reasons why a variance should be granted.


(B)

Authority to Grant Variances.  The Superintendent shall decide whether a variance is authorized for each provision of this Article identified in the variance request on an individual basis.


(C)

Conditions for Granting of Variance.  The Superintendent may authorize a variance only if the utility requesting the variance has demonstrated that:

(1)
One or more conditions not under the control of the utility (such as terrain features or an irregular right-of-way line) create a special hardship that would make enforcement of the provision unreasonable, given the public purposes to be achieved by the provision; and

(2)
All other designs, methods, materials, locations or facilities that would conform with the provision from which a variance is requested are impracticable in relation to the requested approach.


(D)

Additional Conditions for Granting of a Variance.  As a condition for authorizing a variance, the Superintendent may require the utility requesting the variance to meet reasonable standards and conditions that may or may not be expressly contained within this Article but which carry out the purposes of this Article.


(E)

Right to Appeal.  Any utility aggrieved by any order, requirement, decision or determination, including denial of a variance, made by the Superintendent under the provisions of this Article shall have the right to appeal to the Village Board, or such other board or commission as it may designate.  The application for appeal shall be submitted in writing to the Village Clerk within thirty (30) days after the date of such order, requirement, decision or determination.  The Village Board shall commence its consideration of the appeal at the Board’s next regularly scheduled meeting occurring at least seven (7) days after the filing of the appeal.  The Village Board shall timely decide the appeal.


33-4-22
PENALTIES.  Any person who violates, disobeys, omits, neglects or refuses to comply with any of the provisions of this Article shall be subject to fine in accordance with the penalty provisions of this Code.  There may be times when the Village will incur delay or other costs, including third party claims, because the utility will not or cannot perform its duties under its permit and this Article.  Unless the utility shows that another allocation of the cost of undertaking the requested action is appropriate, the utility shall bear the Village’s costs of damages and its costs of installing, maintaining, modifying, relocating, or removing the facility that is the subject of the permit.  No other administrative agency or commission may review or overrule a permit related cost apportionment of the Village.  Sanctions may be imposed upon a utility that does not pay the costs apportioned to it.  (See Section 1-1-20 for additional penalties.)

33-4-23
ENFORCEMENT.  Nothing in this Article shall be construed as limiting any additional or further remedies that the Village may have for enforcement of this Article.

ARTICLE V - STREET IMPROVEMENTS
DIVISION I – CURBS AND GUTTERS


33-5-1
REQUEST IN WRITING.  Any person owning property within the Village who desires to have new curbs and gutters constructed along the street adjoining his premises shall file a request with the Street Superintendent, giving the location of the property and the length of the curbs and gutters requested.

33-5-2
COST TO OWNER.  If the funds are available and the Village Board approves the request, the property owner shall pay one-half (1/2) of the cost of the construction including engineering fees, and thereafter, the curbs and gutters shall be maintained by the Village.

33-5-3
APPROVAL BY VILLAGE BOARD.  The approval of the request for construction of curbs and gutters by the Village Board shall be dependent upon the approval of funds, priority of projects and continuity of construction for the best benefit of the Village as determined by the Village Board.


33-5-4
SUBDIVISIONS.  This Section is not applicable to new subdivisions.  (See 65 ILCS Sec. 5/11-80-13)

33-5-5  -  33-5-9
RESERVED.
DIVISION II – STORM SEWERS

33-5-10
DESCRIPTION OF STORM WATER SEWERS.  Storm water sewers shall be any pipe or sewer used for the carrying of surface drains, ground waters, roof leaders, or storm waters, rain waters, or other waters other than sanitary sewage.

33-5-11
SUPERVISION.  The Superintendent of the Water and Sewer Departments shall supervise all connections made to the public storm sewer system or excavations for the purposes of installing or repairing the same.

33-5-12
PERMITS.  Before any connection is made to the public storm water sewers, a permit shall be applied for and approved by the Superintendent or his designated representative.

33-5-13
REQUIREMENTS: USE OF STORM WATER SEWERS.  Where a storm water sewer is accessible in a street, alley or easement to a building or premises abutting thereon, the surface drains, ground waters, roof leaders, or storm waters shall be discharged into the storm water sewer unless otherwise authorized by the Village.  Under no conditions shall sanitary sewage or wastes or any substance other than surface waters, ground waters, roof waters or storm waters be permitted to flow into or be connected to the storm water sewer; and no sanitary sewer shall be connected to the storm water sewer system.

33-5-14  -  33-5-15

RESERVED.
(See 65 ILCS Sec. 5/11-80-7)
DIVISION III – DRIVEWAYS AND CULVERTS

33-5-16
PERMIT REQUIRED.

(A)

No person shall hereafter construct, build, establish, replace or maintain a driveway or driveway culvert pipe in a street right-of-way maintained by the Village without first obtaining a permit to do so, as hereinafter provided.  Residents may obtain the application and permit for driveway and/or culvert construction or replacement forms from the Village Building Inspector or his or her authorized deputy.


(B)

No work shall be undertaken on Village right-of-way until the approved permit has been received by the applicant.


33-5-17
APPLICATIONS AND PROCEDURE.

(A)

The application for driveway and/or culvert construction or replacement permit on the form supplied by the Village shall be submitted in duplicate to the Building Inspector or his or her authorized deputy along with the required fee.  The application shall be complete and shall include a plan, drawing or sketch of the proposed driveway and/or culvert and the name, address and telephone number of the applicant and of the owner of record of the property to be served by the proposed driveway.

(B)

Within ten (10) days of the receipt of the application, the Building Inspector shall review the proposed driveway and/or culvert and determine whether the proposed driveway conforms to the requirements of this Division.

(C)

If the Building Inspector, or his or her authorized deputy, determines that the project as described in the application conforms to the requirements of this Division, then and in that event a permit for driveway and/or culvert construction shall be issued.

(D)

If the Building Inspector, or his or her authorized deputy, determines that the proposed project does not conform to the requirements of this Division, then and in that event, the Building Inspector, or his or her authorized deputy, shall deny the application and return one (1) copy of the application with a written statement of the reason for the rejection.

(E)

If the application is denied, the applicant may, within fourteen (14) days of the date of the denial, appeal the denial to the Board of Trustees of the Village.  The applicant shall file an original and seven (7) copies of a brief statement of the grounds for his or her appeal in writing with the Village Clerk.

(F)

Upon the filing of an appeal, the Clerk shall notify the President and members of the Board of Trustees of the pending appeal and shall provide each with a copy of the written statement.  Within fourteen (14) days of the filing of an appeal, the Street and Alleys Committee of the Board of Trustees shall convene for the purpose of conducting a hearing at which the Building Inspector, or his or her authorized deputy, and the applicant may be heard as to the denial.  After the hearing, the Committee shall report to the next regularly scheduled meeting of the Board of Trustees as to the recommended disposition of the appeal.

(G)

Upon a majority vote of the Board of Trustees, the Board may affirm the denial, reverse the denial and order the issuance of the permit or vary the requirements of this Division and order the issuance of the permit as the conditions and facts of the case may require.


(H)

Nonrefundable application fees of Ten Dollars ($10.00) shall accompany each application.  No permit shall be issued until the fee is paid in full.

33-5-18
CONSTRUCTION COST.  A driveway and/or driveway culvert constructed under a permit must be done by or for the property owner at his or her expense.  Where a commercial driveway requires additions to the roadway facility, these must also be done at the owner’s expense.  Existing driveways may be altered by the Village, at its expense, when reconstruction or changing conditions warrant.

33-5-19
MAINTENANCE OF DRIVEWAYS.  Property owners having access to a Village right-of-way are fully responsible for the maintenance of their driveway.  This maintenance responsibility includes the removal of snow and ice and keeping the portion of the driveway within the Village right-of-way in a safe condition for the general public.

33-5-20
DRAINAGE, CONSTRUCTION AND CULVERT REQUIREMENTS.

(A)

Driveways must be constructed so that they do not adversely affect the street drainage or drainage of the adjacent property.  The drainage and the stability of the subgrade must not be impaired by driveway construction or roadside development.  In no case may the construction of a driveway cause water to flow across the street pavement, to pond on the shoulders or in the ditch or result in erosion within the right-of-way.


(B)

Drainage collected by ditches, gutters or pipes on private property shall not be discharged into the street drainage system unless expressly approved by the Village.  The permittee may be required to submit a drainage study to the Village justifying the drainage system proposed and the pipe or sewer sizes to be used.  Natural drainage laws and practices must be observed.


(C)

Where the construction of a driveway necessitates crossing a roadway ditch, a culvert pipe shall be installed in the ditch by the permittee.  The low point of the driveway profile shall be at or close to the ditch line.  Under no circumstances will existing ditches or gutters be filled without adequate alternate provisions for drainage being made.


(D)

Culvert pipe shall be of a size adequate to carry the anticipated flow in the ditch and shall not be smaller than twelve (12) inches inside diameter.

(E)

The structural material and gauge of the driveway culvert pipe shall be adequate to withstand loads from the anticipated vehicular traffic across the driveway.


(F)

The culvert shall meet the requirements of the Illinois Standard Specifications for Road and Bridge Construction.


(G)

The length of the culvert may be determined as the sum of the width of the driveway (surfaced width and shoulder) at the ditch line and the length needed to accommodate a sideslope of at least two horizontal to one vertical from the driveway grade to the ditch, with a minimum length of twenty-four (24) feet regardless of ditch depth.

(H)

Flow line of the culvert shall match the flow line of the ditch.  Prior to installation of the culvert pipe, the ditch shall be graded and shaped.


(I)

There shall be a minimum of six (6) inches of cover, including the driveway surface, over the culvert pipe.


(J)

Culvert piping materials must be one of the following:

(1)
Galvanized, corrugated steel culvert pipe: minimum gauge, 16.

(2)
Precoated galvanized corrugated steel culvert pipe.

(3)
Reinforced concrete culvert pipe, storm drain and sewer.


33-5-21
DRIVEWAY STANDARDS.  All driveways hereafter constructed, replaced, established or maintained shall conform with the following:

(A)

Minimum Width.
(1)
Residential zones property = 12 feet.
(2)
Commercial, industrial or agricultural zoned property:
(a)
One-way traffic = 15 feet.
(b)
Two-way traffic = 24 feet.

(B)

Maximum Width.
(1)
Residential:
(a)
Single driveway = 24 feet.
(b)
Combined driveway to serve adjacent lots = 26 feet.
(2)
Commercial, industrial or agricultural = 35 feet.

(C)

Minimum Entrance Radius Flare = 10 feet.

(D)

Maximum Entrance Radius Flare = 30 feet.

(E)

Minimum Distance Between Adjacent Driveways = 10 feet; except for a combined serving adjacent lots.

(F)

Driveway Surface with Street Right-of-Way (over compacted subgrade).
(1)
Crushed stone: minimum thickness = 6 inches.
(2)
Portland cement concrete: minimum thickness = 6 inches.
(3)
Bituminous concrete: no required minimum thickness of bituminous concrete surface but it must be constructed over a compacted crushed stone base not less than 6 inches in thickness.

(G)

Side Slope.  Maximum, 2 horizontal; 1 vertical.

33-5-22
CONSTRUCTION TIMETABLE.  All construction shall be completed within a reasonable time after commencement, and upon completion the applicant shall sow grass seed or take such other measures as may be appropriate to protect the right-of-way.  If in the opinion of the Building Inspector the work is not completed within a reasonable time, the Building Inspector shall, by written notice, give the applicant thirty (30) days to complete the work.  If the work is not completed within that time, the Village may complete the work and the applicant and/or owner shall reimburse the Village for costs incurred.

33-5-23
INSPECTION.  When construction is complete, the applicant shall notify the Building Inspector that the work is ready for inspection.  The inspection will be conducted by the Building Inspector or other person authorized by the Board of Trustees.  Should the work be found unacceptable, the applicant shall have thirty (30) days within which to upgrade the construction to meet the requirements of this Division.  If at the end of that time the work is still unacceptable, the Village may complete the work, and the applicant and/or owner shall reimburse the Village of costs incurred.

33-5-24
LIEN.

(A)

The cost of any work performed by or on behalf of the Village, pursuant to Sections 33-5-22 and 33-5-23, shall be a lien upon the real estate served by the driveway.  Whenever a bill for the work remains unpaid thirty (30) days after it has been 

rendered to the property owner, the Village Clerk shall file within sixty (60) days, after the cost and expense of the work is incurred, with the Recorder of Deeds of Piatt County, Illinois a notice of lien.  The notice shall consist of a sworn statement setting out:

(1)
A description of the real estate sufficient for identification thereof;

(2)
The amount of money representing the cost and expense incurred or payable for the work;

(3)
The date or dates when the cost and expense were incurred by the Village; and

(4)
That the Village is asserting a lien for such cost and expense, pursuant to this Division.


(B)

A copy of the notice shall be mailed by the Village Clerk to the owner of the premises at his or her last known address, if his or her address is known to the Clerk, otherwise to the owner at the address of the premises.

(C)

Property subject to a lien for the unpaid bills shall be sold for nonpayment of the same, and the proceeds of the sale shall be applied to pay the charges, after deducting costs, including reasonable attorney’s fees, as in the case in the foreclosure of statutory liens.  The Village Attorney is authorized to institute the proceedings in the name of the Village in any court having jurisdiction over such matters, within two (2) years from the date the costs and expenses were incurred.


(D)

In addition to the above lien, the Village may file a lien and proceed against the property served by the driveway in accordance with and under the provisions of the Illinois Mechanic’s Lien Act or avail itself of any other legal remedy.

33-5-25
COST OF CONSTRUCTION, OPERATION AND MAINTENANCE.  The applicant shall bear the cost of furnishing all necessary construction, labor, materials and related items necessary to comply with this Division.  Maintenance of the driveway and the costs thereof shall be the responsibility of the owner of the property served by the driveway.  The maintenance of the culvert, other than replacement, which shall be the responsibility of the owner, shall be the responsibility of the Village.  The owner shall at all times maintain that portion of the driveway within the Village right-of-way in a reasonable condition.

33-5-26
VILLAGE GUARANTEES.  While requiring minimum standards for driveways and culvert construction, the Village in no way guarantees that these minimum standards are adequate for the intended use of the improvements.  The Village further reserves the right to waive any of the above requirements if it is determined by the Board of Trustees to be in the best interest of the Village.

33-5-27
VARIANCES.  Whenever unusual conditions prevent the strict enforcement of these regulations, or as a practical matter, the Village Board, by resolution, may vary or modify them in such a way that the permit will be granted.

(See 65 ILCS 5/11-80-7)

(Ord. No. 90-2; 04-02-90)

ARTICLE VI - SNOW REMOVAL


33-6-1

DEFINITIONS.  The following definitions shall apply in the interpretation and enforcement of this Article.

“BUSINESS DAY” is any day not a Sunday or a National Holiday.

“BUSINESS DISTRICT” shall include all those areas zoned for business, commercial and industrial purposes in the Zoning Code and accompanying map as amended or other applicable regulations.

“BUSINESS HOURS” are the hours between 8:00 A.M. and 5:00 P.M. on any business day.

“ROADWAY” means that portion of a street or highway improved, designed, or ordinarily used for vehicular travel, exclusive of the berm or shoulder.

“SIDEWALK” means that portion of a street between the curb lines or the lateral lines of a roadway and the adjacent property lines intended for the use of pedestrians.

“STREET” OR “HIGHWAY” means the entire width between the boundary lines of every way publicly maintained when any part thereof is open to the use of the public for purposes of vehicular travel.


33-6-2

SNOW AND ICE TO BE REMOVED FROM SIDEWALKS BY PRIVATE PERSONS.


(A)

Every person in charge or control of any building or lot of land within the Village fronting or abutting on a paved sidewalk, whether as owner, tenant, occupant, lessee or otherwise, shall remove and clear away, or cause to be removed and cleared away, snow and ice from a path from so much of a sidewalk as is in front or abuts on said building or lot of land.  Snow and ice shall be so removed from sidewalks in all business districts within the Village by twenty-four (24) hours after cessation of any fall of snow, sleet or freezing rain.  The path required to be cleared in the business district shall be six (6) feet in width, or the whole width of the sidewalk, whichever is smaller.


(B)

However, in the event snow and ice on a sidewalk has become so hard that it cannot be removed without likelihood of damage to the sidewalk, the person charged with its removal shall, within the time mentioned in subsection (A) above, cause enough sand, cinders or other abrasive to be put on the sidewalk to make travel thereon reasonably safe; and shall then, as soon thereafter as weather permits, cause a path on said sidewalk of at least six (6) feet in width to be thoroughly cleaned.


33-6-3

DEPOSITING OF SNOW AND ICE RESTRICTED.  No person shall deposit or cause to be deposited any snow and ice on or against a fire hydrant or on any sidewalk, roadway, or loading or unloading areas of a public transportation system, except that snow and ice may be windrowed on curbs incident to the cleaning of sidewalks in business districts.  (See 65 ILCS Sec. 5/11-80-13)


33-6-4

MAYOR’S AUTHORITY.  The Mayor is hereby authorized on behalf of the Village to cause sidewalks to be cleared upon the request of the person or entity charged with snow and ice removal.  Any person or entity whose sidewalk is cleared pursuant to this Section shall reimburse the Village for the costs of such clearing.

ARTICLE VII - MOVING BUILDINGS


33-7-1
PERMIT REQUIRED.  It shall be unlawful for any person to move or cause to be moved, any building in, into, through, or from the Village without first obtaining a permit therefor from the Village Clerk.  Such permit shall be known as a “House Moving Permit”.


33-7-2
APPLICATION FOR PERMIT.  Any person desiring such a permit shall file with the Village Clerk an application therefor in writing on a form to be furnished by the Street Superintendent for that purpose.  Such application shall specify the following:


(A)

The character and size of the building to be moved;


(B)

The reason for such moving;


(C)

The use, purpose and occupancy for which said building or structure is to be used;


(D)

The location from which and to which said building is to be moved;


(E)

A plot plan showing the proposed location of the building upon the property to which said building is to be moved, provided said location is in the Village;


(F)

The streets on, over or through which it is desired to move said building;


(G)

Whether the building conforms to the Zoning Code or other applicable regulations in the location to which it is to be moved.


33-7-3
INVESTIGATION.  Upon the filing of the application, the Street Superintendent shall cause the Zoning Administrator, or other authorized representative of the Village, to investigate the building and report to him the results of such investigation, together with recommended action thereon.


33-7-4
DENIAL OF PERMIT.  No person shall be issued to move any building or structure which, in the opinion of the Street Superintendent:


(A)

Is so constructed or in such condition as to be dangerous;


(B)

Is infested with pests or unsanitary;


(C)

If it is a dwelling or habitation, is unfit for human habitation;


(D)

Is so dilapidated, defective, unsightly, or in such a condition of deterioration or disrepair that its relocation at the proposed site would cause appreciable harm to or be materially detrimental to the property or improvements in the district within a radius of one thousand (1,000) feet from the proposed site;


(E)

If the proposed use is prohibited by the Zoning laws of the Village;


(F)

If the structure is of a type prohibited at the proposed location by any ordinance of the Village; or


(G)

If the moving of the building or structure causes unreasonable damage to the trees, plants and/or shrubs on and along the public streets.


Provided, however, that if the condition of the building or structure, in the judgment of the Street Superintendent, admits of practicable and effective repair, the permit may be issued upon the terms and conditions as set forth herein.


33-7-5
TERMS AND CONDITIONS OF PERMIT.  When a house moving permit is granted such terms and conditions as may be deemed reasonable and proper may be imposed, including, but not limited to the public streets or other public property in the Village on, over, or through which the building or structure shall be moved, and the requirements of changes, alterations, additions or repairs to be made to or upon the building or structure, to the end that the relocation thereof will not be materially detrimental or injurious to public safety or to public welfare or to the property and improvements, or either, in the district to which it is to be moved.  Such terms and conditions shall be written upon the permit or appended in writing thereto.


33-7-6
ESTIMATE OF COST AND DEPOSIT.  The applicant shall also deposit with the Village Clerk a cash deposit sufficient to cover the cost to the Village as estimated by the Street Superintendent of trimming, moving, removing or replanting of trees or shrubs, and of moving, removing and displacing any pole or other structure, supporting any wires, cables or other equipment belonging to the Village or the cutting, displacing or changing the location of any wire, cable or other equipment upon said poles or structures belonging to the Village.


33-7-7
LIABILITY INSURANCE.  Every person moving a building in the Village shall file with the Village Clerk a liability insurance policy issued by the solvent corporation holding a certificate of authority to do insurance business in the State, which policy shall conform in all respects to the requirements of this Section.


In lieu of filing the insurance policy herein referred to, a certificate of insurance issued by an insurance corporation may be filed.  The certificate must show that a policy meeting the requirements of this Section has been issued, and shall set forth the expiration date of said policy.


The liability policy required under this Section shall insure the person moving a building against loss from the liability imposed by law for injury to, or death of any person, or damage to any property growing out of the moving of such building to the amount or limit of Fifty Thousand Dollars ($50,000.00) exclusive of interest and costs, on account of injury to or death of any one (1) person, of One Hundred Thousand Dollars ($100,000.00) exclusive of interest and costs, on account of moving any one (1) building resulting in injury to or death of more than one (1) person, and of Twenty-Five Thousand Dollars ($25,000.00) for damage to property of others, resulting from moving any one (1) building.


33-7-8
OWNER’S COMPLETION BOND OR SAVINGS AND LOAN CERTIFICATE AND SHARE.  Prior to the issuance of a permit to move a building, the owner or lessee of the property upon which the building is to be located shall file with the Zoning Administrator or other authorized representative of the Village, a corporate surety bond, conditioned as follows:


That all of the work required to be done to complete the relocation, alteration and reconstruction of the building pursuant to the conditions of the permit shall be fully performed and completed within a reasonable time to be specified in the permit by the Zoning Administrator, or other authorized representative of the Village.  Such bond shall be in principal amount equal to the estimated cost of the work proposed to be done, plus ten percent 

(10%) thereof, and shall name the Village as obligee, and shall be in a form approved by the Village Attorney.


In lieu of furnishing such a corporate surety bond, the owner or lessee may post a cash deposit in the amount of said bond.


An extension of time for the completion may be granted in writing by the Zoning Administrator or other authorized representative of the Village when, in his discretion, circumstances shall so justify, but no such extension shall release any surety or other security.


33-7-9
CLEARANCE OF SITE AND SAFETY MEASURES REQUIRED.  Prior to the issuance of a permit to move a building, the owner or lessee of the property from which the building is to be moved shall file with the Village Clerk a bond or other form of security in favor of the Village, conditioned as follows:


(A)

Before any work is started on a building or structure, the permittee or his authorized agent shall notify the appropriate utilities in order that all gas, water and oil pipelines that are to be disconnected from the building may be securely capped and sealed.


(B)

Immediately, after the moving of any building or structure, the permittee or his authorized agent shall securely barricade all basement excavations and other holes or openings.


(C)

Within ten (10) days after the moving of any building or structure, the permittee or his authorized agent shall complete the following work:

(1)
Securely close and seal any sanitary piping located on the property.

(2)
Fill with dirt or sand any septic tanks or cesspools located on the property.

(3)
Fill any openings, excavations or basements remaining on the land with dirt or sand to street level or the natural level of adjoining property, unless otherwise directed by the Street Superintendent.

(4)
Remove any buried underground tanks formerly used for storage of flammable liquids.

(5)
Removal all refuse, debris and waste materials from the property.



The bond required by this Section shall be an amount equal to the cost of the work proposed to be done, as estimated by the Street Superintendent.


The bond may be in the form of a corporate surety bond, cash deposit, savings and loan certificate, or an instrument of credit.


An extension of time for completion of the work required by this Section may be granted by the Street Superintendent when, in his discretion, circumstances justify such an extension; but no such extension shall release any bond or other security furnished pursuant to this Section.


33-7-10
INSPECTION FEE AND PERMIT FEE.  An inspection fee in the sum of Fifteen Dollars ($15.00) shall be paid to the Village Clerk upon filing of each application for a house moving permit.


A permit fee in the sum of Ten Dollars ($10.00) shall be paid to the Village Clerk upon the issuance of each house moving permit.


33-7-11
ISSUANCE OF PERMIT.  The Street Superintendent shall approve the issuance of a house moving permit when all the necessary requirements and conditions of this Article have been complied with.  It shall then be the duty of the Village Clerk to issue the permit.


33-7-12
SUSPENSION OR REVOCATION OF PERMIT.  The Street Superintendent, at any time, and for sufficient cause, may revoke or suspend any permit granted under this Article.


33-7-13
CONTROL AND SUPERVISION.  Every building which is moved on, over, or through any public street, way or park in the Village shall be under the control of the Street Superintendent and every such building shall be moved in a careful manner and the work shall be prosecuted with diligence and to the satisfaction and approval of the Street Superintendent.  This Section in no way relieves the person having charge of the moving of any building of his obligation to furnish proper supervision.


33-7-14
NOTICE REQUIRED.  Notice must be given to both the Street Department and the Police Department of the Village by the person or his representative to whom the permit is issued not less than forty-eight (48) hours nor more than seventy-two (72) hours before the actual work of moving a building or structure is to commence.


33-7-15
DEFAULT IN PERFORMANCE OF CONDITIONS.  Whenever a default has occurred in the performance of any term or condition of any permit, written notice thereof shall be given to the permittee by the Street Superintendent; said notice to state the work to be done, the estimated cost thereof, and the period of time deemed to be reasonably necessary to complete such work.  After receipt of such notice, the permittee must, within the time therein specified, either cause the work to be done, or pay over to the Village Clerk the estimated cost of doing the work, as set forth in the notice, plus ten percent (10%) of said estimated cost.  Upon receipt of notice from the Village Clerk that the permittee has deposited such money, the Street Superintendent shall cause the required work to be performed and completed.


If the permittee defaults, the Village shall have the option in lieu of completing the work required to demolish the building or structure and to clear, clean and restore the site or sites.


33-7-16
APPROVAL OF ROUTE.  The streets over which any building or structure is to be moved must be recommended by the Street Superintendent and the Chief of Police to the Village Board for approval.


33-7-17
OBSTRUCTING STREETS.  No person owning or having charge of the moving of any building into, on, over, through, or from any public streets, ways or parks in the Village shall permit said building to remain in any one location on any such street, way or park for a period longer than twenty-four (24) hours, except by written permission obtained from the Chief of Police, or to obstruct traffic on any railroad.


33-7-18
LIGHTS AND BARRICADES.  The person having charge of the moving of any structure shall maintain proper lights and barricades whenever such structure is on any public street, way or park during the hours of darkness.


33-7-19
WIRES AND STRUCTURAL SUPPORTS.  In the event that the moving of any building for which a permit shall have been granted hereunder makes it necessary to move, remove or displace any pole or other structure supporting the wires, cables or other equipment of any public utility or of the Village, or to cut, displace or change the location of any wire, cable or other equipment upon said pole or structure, the person to whom such permit has been granted, or his authorized representative, shall obtain permission in writing from the owner or owners of such pole, structure or wires, cables or other equipment thereon, and shall notify such owner or owners at least seventy-two (72) hours prior to the time that the moving of such building will necessitate the removal of such obstructions.


The person to whom the permit is granted shall not, at the expiration of said time of notice or at any time, cut, move or in any way disturb such public utility or Village property; and such work shall be done only by the authorized workmen of the utility or the Village, whichever is the owner.


The person to whom the permit is granted shall pay to the public utility, or to the Village, as the case may be, any and all costs or expense for the removal, rearrangement or replacement of any pole or structural support of wires, cables or equipment thereon or of any damage to such property.


33-7-20
TREES, PLANTS AND SHRUBS.  In the event that the moving of any building for which a permit shall have been granted hereunder makes it necessary to trim, move, remove or replant any tree, plant or shrub belonging to or under the control of the Village, the person to whom such permit has been granted or his authorized representative shall notify the Street Superintendent at least seventy-two (72) hours prior to the time that the moving of such building will necessitate the removal of such obstructions.


The person to whom the permit is granted shall not, at the expiration of the time of notice, or at any time, trim, move, remove, replant, or otherwise disturb such trees, plants or shrubs; and such work shall be done only by the authorized workmen of the Village unless otherwise approved and so ordered by the Street Superintendent.


The person to whom the permit is granted shall pay to the Village, any and all costs or expenses for the trimming, moving, removing or replanting of any trees, plants or shrubs or of any damage thereto.


33-7-21
REPAIRS TO PUBLIC PROPERTY.  In the event that the moving of any building for which a permit shall have been granted hereunder causes damage to the public streets or other public property, in addition to any other remedies the Village may have, the Street Superintendent may cause such damage to be repaired and the cost thereof shall be deducted from the deposit required herein, or he may require the person to whom such a permit has been granted, or his authorized representative, upon written notification from the Street Superintendent to make all necessary repairs to such streets or property; provided, however, that should the person to whom the permit has been granted and to whom the notice 

has been given, or his authorized representative, fails to make the necessary repairs within the period of time designated in the written notice, the Street Superintendent may cause such necessary repairs to be made and the cost thereof deducted from the deposit required herein.


33-7-22
REFUNDING OF DEPOSITS.  When the moving of any building for which a permit has been granted is completed, and all damage to public streets or other public property has been repaired to the satisfaction of the Street Superintendent, and all costs of repairing damages or performing other work as provided herein have been paid and the deposit as required by Section 33-7-6, or such portion thereof then remaining unused under the provisions of this Article, shall be refunded upon surrender of the deposit receipt representing the money so deposited.  However, should the cost of repairing damages and/or performing other work, as in this Article provided, exceed the total amount of money deposited, the person to whom the permit was granted shall be held liable for the amount of damage and/or other costs which are in excess of the amount deposited.  It shall be the duty of the Village Clerk, upon receipt of the request from the Street Superintendent, to collect such part of the claim which is in excess of the deposit from the person to whom the permit was granted.
ARTICLE VIII – DRAINAGE SYSTEM


33-8-1
PERMIT REQUIRED.  All persons with property located within the Village seeking to connect their property to the Village drainage tile system including a catch basin, must have an approved permit before making such connection and pay a Twenty-Five Dollar ($25.00) non-refundable permit fee.


33-8-2
CONNECTION FEE.  There shall be a Six Hundred Dollar ($600.00) connection fee submitted with the permit application.  If the permit is denied, the connection fee shall be refunded to the applicant.


33-8-3
SUPERVISOR OF THE PUBLIC WORKS DEPARTMENT.  Permit applications shall be reviewed, and approved or denied or modified by the Supervisor of the Public Works Department.


33-8-4
SLEEVE OR COLLAR REQUIRED.  All connections in the Village drainage tile system must use a sleeve or a collar approved by the Village.


33-8-5
WATER SAMPLING REQUIRED; PAYMENT OF WATER SAMPLING.  All connections must contain an inspection port located at an easily accessible location at ground level in order to allow for yearly water sampling.  All persons granted a permit under this Article expressly consents to yearly water sampling by the Village and/or the Village contractors.  All persons that connect to the Village drainage tile system shall pay an annual test fee for this sampling, with said test fee payable by February 1 of each calendar year.  The annual water sampling fee shall be included in the property owner’s water coupon book.  Failure to pay the annual water sampling fee shall subject the property owner to the water shut off provisions pursuant to the Village Water Code.

33-8-6
STREETS AND SIDEWALKS; UNDERGROUND BORING REQUIRED.  No property owner shall break or cut any Village street or sidewalk when connecting to the Village drainage tile system.  Underground boring shall be required if a street or sidewalk must be crossed in order to make a connection.  The underground boring shall be done at the property owner’s expense.  The property owner shall be liable to the Village for any damage done to any street or sidewalk due to underground boring.

33-8-7
WAIVER OF LIABILITY.  All property owners granted a permit under this Article expressly waive, and hereby hold the Village harmless for any and all claims and damages occurring in the past, present or future, that said property owners may have against the Village, regardless of the fault of the Village, arising from the condition, or performance, of the Village drainage tile system.

33-8-8
PERMIT APPLICATION.  The Supervisor of the Public Works Department shall be responsible for creating a permit application that conforms with the intent of this Article.  Said applications shall be available at the office of the Village Clerk.

33-8-9
ALL WORK DONE IN CONFORMANCE WITH APPLICABLE LAW.  Any person desiring to connect to the Village drainage tile system shall use licensed contractors to perform the connection.  All connections must be done in accordance with applicable federal, state, county and local law and comply with any existing federal, state, county, and local permitting process.

33-8-10
APPEAL TO VILLAGE BOARD.  If the Supervisor of the Public Works Department denies an application for a permit, the aggrieved property owner may appeal the Supervisor’s decision to the Village Board by timely placing the issue of the denial upon the agenda for the next regularly scheduled Village Board meeting.
(Ord. No. 2018-02; 04-02-18)

(65 ILCS 5/11-109-1)
33-49
[Supplement No. 3; 09-01-18]

